Checkpoint Requirements

and Planning Guide (CRPG)

August 1, 2023

33

Notice

This document is distributed under the sponsorship of the Transportation Security Administration (TSA) of the U.S. Department of Homeland Security in the interest of information exchange. The U.S. Government assumes no liability for the contents or use.

Note: The Checkpoint Requirements and Planning Guide (CRPG) consists of the main document (what you are currently reading) and three appendices. The main document is available here: https://www.tsa.gov/sites/default/files/checkpoint-requirements-and-planning-guide.docx.

Appendix A contains the equipment blocks and tags and is available here: https://www.tsa.gov/sites/default/files/checkpoint-requirements-and-planning-guide_appendix-a.docx.

Appendix B contains the electrical and data information and is available here: https://www.tsa.gov/sites/default/files/checkpoint-requirements-and-planning-guide_appendix-b.docx.

Appendix C contains checkpoint layout examples and is available here: https://www.tsa.gov/sites/default/files/checkpoint-requirements-and-planning-guide_appendix-c.docx.

We have made the main CRPG Section 508 compliant. We are working to make the embedded files and appendices 508 compliant. Non-compliant documents will be replaced as soon as possible.

| Helpful Instructions | Accessing Embedded Files |  |
| --- | --- | --- |

Disclaimer

It is the project sponsor’s responsibility to create drawing concepts, issue construction documents, and provide record as-built drawings in AutoCAD and PDF. Maximum flexibility in the checkpoint size and infrastructure must be considered for future checkpoint reconfigurations and new checkpoint terminal planning. TSA equipment is subject to change, which may affect the checkpoint space and infrastructure requirements. Additionally, adjustments may be required after installation of TSA equipment if there are changes to design and/or operations of the checkpoint. The project sponsor should be prepared to update these documents as needed.

Prior to beginning and finalizing checkpoint designs, airports should check with TSA to confirm they have the latest Checkpoint Requirements and Planning Guide standards. The CRPG is subject to change on a case-by-case basis for equipment layout and components of the checkpoint and may include addition or removal of items listed/not listed within this document. Users of this document should check with Dale Mason (dale.mason@tsa.dhs.gov) from the Requirements Development Branch for the latest documents/standards and/or obtain the updated documents/standards directly from tsa.gov at the links provided above.

TSA does not endorse specific equipment or specific original equipment manufacturers. All equipment shown is for example only.

This guide is dynamic and easily accessed from public domains for all who need to create airport checkpoint drawings and projects. Resource and requirement documents that are embedded in the PDF can be accessed and studied for standard and complex facets of any checkpoint project.

Version History

| Revision | Date | Description of Changes | Sections Affected |
| --- | --- | --- | --- |
| 0 | 12/17/2018 | Initial Publication | All |
| 1 | 01/06/2020 | 2020 Edition Updates | All |
| 2 | 09/30/2021 | 2021 Edition Updates | All (Change Descriptions in Each Section) |
| 3 | 09/30/2022 | 2022 Edition Updates | Sections 1 through 9 (Change Descriptions in Each Section) |
| 4 | 08/01/2023 | 2023 Edition Updates | Updated main document for 508 compliance; Terminology Table updated to add Open Architecture (OA), Security Operations (SO), One Stop Program (OSS), SoS (System of Systems), to remove BAT (Biometric Authentication Technology), eAIT (enhanced Advanced Imaging technology), EMD (Enhanced Metal Detector), RR (Resolution Room), and to update APSS to reflect Accessible Property Screening System and account for CT/CPSS in 1-to-1 and 2-to-1; Sections 2 through 10 updated (Change Descriptions in Each Section); Electrical and Data (former Section 7) moved into separate Appendix B; Pulled equipment blocks and tags and electrical device configuration sheets into separate Appendix A; Moved checkpoint layouts to a new Appendix C |

Table of Contents

Notice	i

Disclaimer	i

Version History	ii

Table of Contents	iii

Security Screening Checkpoint Terminology	v

1	Introduction	1-1

1-1	Purpose	1-2

1-2	Scope	1-2

2	Roles, Responsibilities, and Project Planning	2-1

2-1	Roles and Responsibilities	2-2

2-2	Project Planning and Design	2-6

3	Security Screening Checkpoints	3-1

3-1	SSCP Elements	3-2

3-2	Transportation Security Equipment	3-2

3-3	Planning Considerations for Emerging Capabilities	3-3

3-4	SSCP Boundaries	3-3

3-5	Exits from Secure Side	3-5

3-6	Known Crewmember®	3-6

3-7	Federal Inspection Service (FIS) Checkpoints	3-7

3-8	Checkpoint Branding	3-10

4	Checkpoint Approach and Passenger Queue	4-1

4-1	Pre-Screening Preparation Instruction Zone	4-2

4-2	Passenger Queue	4-4

4-3	TDC and CAT-1/2	4-7

5	Passenger and Carry-On Baggage Screening	5-1

5-1	SSCP Layouts	5-2

5-2	Advanced Technology	5-4

5-3	Computed Tomography	5-5

5-4	Automated Screening Lanes (ASLs)	5-8

5-5	Walk-Through Metal Detector (WTMD)	5-8

5-6	Advanced Imaging Technology	5-10

5-7	Divest Table/Divest Automation	5-14

5-8	Barriers	5-14

5-9	Americans with Disabilities Act/Access Gate	5-15

5-10	Diverter Roller	5-15

5-11	Buffers/Decision Points	5-16

5-12	Composure	5-16

5-13	Bins, Rerun Carts and Bin Return	5-17

5-14	Secondary Screening	5-17

5-15	Bag Search Table	5-19

5-16	Passenger Inspection	5-20

5-17	Composure Bench	5-20

5-18	Supervisor Podium	5-20

5-19	Equipment Dimensional Criteria	5-21

6	Emerging Technology (TSA Certified Transportation Security Equipment)	6-1

6-1	Passenger Authentication	6-2

7	Supplemental Information	7-1

7-1	Emergency Checkpoints	7-2

7-2	Closed-Circuit Television (CCTV) Requirements	7-2

7-3	Safety Requirements	7-3

7-4	Security Shielding	7-4

8	Structural Considerations	8-1

8-1	Structural Overview	8-2

8-2	Seismic Anchoring, Fire Codes, and Other Local Codes	8-2

9	Equipment Design and Engineering Requirements in the TSA Checkpoint Environment (Future Equipment Vendor Submissions)	9-1

9-1	Equipment Length and Width Considerations	9-2

9-2	Equipment Maintenance Zones	9-2

9-3	Ceiling Height Limitations	9-3

9-4	Equipment Interference Requirements	9-3

9-5	Structural Analysis	9-3

9-6	Sloped Floor Requirements	9-3

9-7	Equipment Geometry and Floor Load Drawings	9-3

9-8	Equipment Power and Data Coordination Drawings	9-4

9-9	Open Architecture	9-4

10	Closing and Links to Appendices	10-1

Security Screening Checkpoint Terminology

| Acronym | Definition |
| --- | --- |
| 1-to-1 | 1 AT/CT/CPSS for 1 WTMD and 1 AT/CT/CPSS for 1 AIT |
| 2-to-1 | 2 AT/CT/CPSS for 1 WTMD 2 AT/CT/CPSS for 1 AIT |
| 3D | Three Dimensional |
| A | Amps |
| A4A | Airlines for America |
| A&E | Architectural and engineering |
| ABC | Alarm Bag Cutout |
| aCAD | AutoCAD |
| ACG | Area Control Gateway |
| ACL | Acceptable Capability List |
| ADA | Americans with Disabilities Act |
| AFF | Above Finished Floor |
| AFSD | Assistant Federal Security Director |
| AHJ | Authority Having Jurisdiction |
| AIT | Advanced imaging technology |
| ALPA | Air Line Pilots Association, International |
| ANSI | American National Standards Institute |
| AOR | Architect of Record |
| APSS | Accessible Property Screening System |
| ARW | Alarm Resolution Workstation |
| ASHRAE | American Society of Heating, Refrigerating and Air Conditioning Engineers |
| ASL | Automated Screening Lane |
| ASP | Airport Security Plan |
| AT | Advanced Technology |
| AT2 | Second Deployment of AT; includes AVS |
| ATR | Automatic Target Recognition |
| ATSA | Aviation and Transportation Security Act |
| AVS | Alternate Viewing Station |
| BLS | Bottled Liquids Scanner |
| BRS | Bin Return System |
| BU | Buffer Power Conveyor |
| CAD | Computer-aided design |
| CAMB | Capabilities Acceptance Management Branch |
| CAP | Capability Acceptance Process |
| CAT | Credential Authentication Technology |
| Cat5/Cat5e/Cat6/ Cat 6a | Category 5 data cable/Category 5 enhanced data cable/Category 6 data cable/ Category 6 advanced data cable |
| CBIS | Checked Baggage Inspection System |
| CCTV | Closed-Circuit Television |
| CES | Canine Enhanced Screening |
| CL | Centerline |
| CPSS | Checkpoint Property Screening System |
| CRPG | Checkpoint Requirements and Planning Guide |
| CSS | Checkpoint and Specialized Screening |
| CT | Computed tomography |
| CTI | Canine Training Instructor |
| CWA | Canine Work Area |
| DOT | Department of Transportation |
| DHS | Department of Homeland Security |
| DV | Divest Roller |
| EDCT | Explosives Detection Canine Team |
| EMT | Electrical Metallic Tubing (EMT) |
| EPPM | Environmental Planning Program Manager |
| ERI | Equipment Request Interface |
| eTAS | Electronic Time, Attendance, and Scheduling |
| ETD | Explosive Trace Detection |
| FAA | Federal Aviation Administration |
| fc | Foot-candles; unit of luminance or light intensity |
| FF&E | Furniture, fixtures, and equipment |
| FIS | Federal Inspection Service |
| FDRS | Field Data Recording System |
| FLEx | Future Lane Experience |
| FOC | Full operational capability |
| FRD | Functional Requirements Document |
| FRM | Field Regional Manager |
| FSD | Federal Security Director |
| ft.2 | Square foot (feet) |
| FY | Fiscal Year |
| GRC | Galvanized rigid conduit (GRC) |
| Hi-SOC | High Speed Operational Connectivity |
| HQ | Headquarters |
| HSC | High Speed Conveyor |
| HT | High Threat Containment Unit |
| HVAC | Heating, ventilation, and air conditioning |
| ICM | Input Control Module |
| IDF | Intermediate distribution frame |
| IEEE | Institute of Electrical and Electronics Engineers |
| IESNA | Illuminating Engineering Society of North America |
| IMAC | Install, Move, Add, or Change |
| IO | Image Operations or Image Operator |
| IP | Internet protocol |
| I-SAT | Integration Site Acceptance Test |
| IT | Information Technology |
| KCM | Known Crewmember® |
| kVA | Kilovolt-ampere |
| LAGs | Liquids, Aerosols, and Gels |
| LAN | Local Area Network |
| LCU | Lane Control Unit (for the Leidos ProVision AIT) |
| LEO | Law Enforcement Officer |
| LH | Left-hand |
| LPD | Last Point of Departure |
| MAX | Maximum |
| MCB | Main Circuit Breaker |
| MDF | Main distribution frame |
| MDR | Manual diverter roller |
| MIL-STD | Military Standard |
| MIN | Minimum |
| mod set | Module set |
| NA | Non-Accessible |
| NEC | National Electric Code |
| NEDCTP | National Explosive Detection Canine Team Program |
| NEMA | National Electrical Manufacturers Association |
| NEPA | National Environmental Policy Act |
| OEM | Original equipment manufacturer |
| OOB | Out of Band |
| OQC | Open Queue Configuration |
| OSHA | Occupational Safety and Health Administration |
| OSHE | Occupational Safety, Health, and Environment |
| OSS | One Stop Security |
| PARAS | Program for Applied Research in Airport Security |
| PAX | Passenger |
| PDF | Portable Document Format |
| POC | Point of contact |
| POE | Power Over Ethernet |
| POR | Program of requirements |
| POTS | Plain Old Telephone Service |
| Pre✓® | PreCheck |
| PPE | Personal protective equipment |
| psf | Pounds per square foot |
| PSP | Passenger Screening Program |
| PSR | Private Screening Room |
| PVS | Primary Viewing Station |
| QDP | Qualification Data Package |
| QPS | Quick Personnel Security |
| RDM | Regional Deployment Manager |
| ReMAG | Requirements Management Advisory Group |
| RFP | Request for Proposal |
| REMD | Real Estate Management Division |
| RF | Radio frequency |
| RFID | Radio-frequency identification |
| RGS | Rigid Galvanized Steel |
| RH | Right-hand |
| RJ | Reject Roller |
| R&S | Rohde & Schwarz |
| RSG | ReadySeeGo |
| RV | Revest Roller |
| SC | Security Capabilities |
| SCP | System Control Processor |
| SE | Structural Engineer |
| SI | Systems Integrator |
| SME | Subject matter expert |
| SO | Scanning Operator OR Security Operations (formerly known as Office of Security Operations – OSO) |
| SOP | Standard Operating Procedure |
| SoS | System of Systems |
| SOW | Scope or Statement of Work |
| SSCP | Security Screening Checkpoint |
| SSI | Security Sensitive Information |
| STIP | Security Technology Integrated Program |
| STSO | Supervisory Transportation Security Officer |
| TCOP | Touch Control Operator Panel |
| TIP | Threat Image Projection |
| TDC | Travel Document Checker |
| TLC | TSA Logistics Center |
| TRX | TIP-Ready X-ray |
| TSA | Transportation Security Administration |
| TSE | Transportation Security Equipment |
| TSL | Transportation Security Laboratory |
| TSO | Transportation Security Officer |
| UPS | Uninterruptible power supply |
| UTP | Unshielded Twisted Pair |
| V | Volts |
| VIPR | Visible Intermodal Prevention & Response |
| VoIP | Voice over Internet Protocol |
| WAP | Wireless Access Point |
| WTMD | Walk-Through Metal Detector |

# 1	Introduction

The Transportation Security Administration (TSA) is mandated by law to screen air travelers and their carry-on bags to identify and intercept prohibited items at the Security Screening Checkpoints (SSCPs) at federalized airports across the United States. Airports must supply adequate space in an acceptable location to be able to install the required equipment and meet all TSA standards. Each checkpoint represents a point of entry into the aviation system and must meet security criteria. Aviation security continues to evolve and adapt to meet the constantly changing threat environment. This Checkpoint Requirements and Planning Guide (CRPG) was created to:

Communicate the current checkpoint design standards for qualified technologies

Provide information and planning considerations for emerging technologies anticipated to be qualified within the near future

Provide information to locate equipment within the checkpoint to provide the highest level of security screening and efficiency from the queue through the composure area

| Revision | Date | Description of Changes |
| --- | --- | --- |
| 0 | 12/17/2018 | Initial Publication |
| 1 | 09/30/2021 | Updated Format; Changed “Global Strategies” to “International Operations” in 1-1 |
| 2 | 09/30/2022 | Minor corrections to wording |

## 1-1	Purpose

The information in this guide must be used when designing new checkpoints or reconfiguring existing checkpoints. Any new or reconfigured checkpoint or lane is required to meet the latest standards. Proper SSCP design helps avoid costly problems for the airport, airlines, and TSA. It also provides a smoother, safer experience for the passenger while increasing the efficiency of the screening process. Improper SSCP design can result in terminal and checkpoint queue congestion, decreased efficiencies, flight delays, and unacceptable security risks.

The CRPG can be used to guide airports and countries in development of pre-clearance locations around the world. The CRPG can also be used by International Operations to assist our international partners in raising the security baseline for Last Point of Departure (LPD) flights to the United States by sharing design standards and lessons learned with other countries for the deployment of security technology.

This guide is a “living” document that is updated when new technologies or processes are adopted by TSA. Airports should use the link on the cover page of this guide to obtain the most recent version.

## 1-2	Scope

Every airport and airport terminal building is unique in physical design and functional requirements. A single SSCP solution will not work for every airport. Every SSCP location must be reviewed as an entity of the overall airport security system. The CRPG provides direction and recommendations on how to construct, deploy and install equipment for, locate, and size a new SSCP and plan for future capability based on the following conditions:

Facility infrastructure and operations

Current screening technology/equipment

Type of risk

Passenger loads/number of enplanements/aircraft type

The CRPG is intended to supplement—but not replace—original equipment manufacturer (OEM) requirements and recommendations, as well as local codes and regulations. All designs and reconfigurations must be coordinated with the TSA Headquarters (HQ) checkpoint designer, TSA HQ stakeholders, the local Federal Security Director (FSD) and staff, and local airport stakeholders so that the recommended guidelines can be incorporated into the final checkpoint design.

# 2	Roles, Responsibilities, and Project Planning

SSCPs are an important element to an airport’s overall terminal design and must be considered in the early stages of planning and conceptual layout. Every airport has a unique physical design and functional requirements; therefore, a single design solution will not work for every checkpoint. Frequent communication and close coordination between TSA HQ, local TSA, and the project sponsor are critical to successful construction of a new, or modification of an existing, TSA SSCP. To effectively and efficiently coordinate and design a checkpoint that will maximize security and throughput, reduce operating costs, and improve passenger experience, it is critical to understand the project planning and design process as well as timing of key considerations and dependencies.

| Revision | Date | Description of Changes |
| --- | --- | --- |
| 0 | 12/17/2018 | Initial Publication |
| 1 | 01/02/2020 | CRPG 2020 Edition Updates |
| 2 | 09/30/2021 | Updated Format; Updated to Clarify Roles and Responsibilities; Added Information on Capabilities Acceptance; Updated Link to Latest PARAS Report 0028, Recommended Security Guidelines for Airport Planning, Design, and Construction |
| 3 | 09/30/2022 | Updated Capabilities Management Division to Capabilities Management Branch; Minor Wording Clarifications |
| 4 | 08/01/2023 | Updated the embedded Financial Roles and Responsibilities and NEPA Fact Sheet documents in 2-1; Clarified information regarding the Capabilities Acceptance Management Branch, the facilitation of security screening equipment donations, and added a contact email in 2-1 |

## 2-1	Roles and Responsibilities

TSA has established guidelines, requirements, and resource documents in the CRPG to assist architectural and engineering (A&E) staff in developing an approved and compliant SSCP environment. These standards ensure seamless deployment of approved equipment and resources to support infrastructure updates, enabling heightened security, increased throughput, and reduction of on-the-job injuries; more dynamic staffing; adherence to federal requirements and improved passenger experience. Involvement of and communication across the stakeholders are crucial to confirm adherence to the TSA design standards. These are subject to change, and the latest information should be gleaned early on.

The number of lanes approved for deployment on any project is driven by a capacity analysis at TSA HQ through the Requirements Management Advisory Group (ReMAG).

On all projects, the TSA Checkpoint Design Team is the representative from TSA responsible for review and approval of checkpoint design submittals. Any work done prior to a TSA approved concept design is done at risk. Once a concept meets TSA requirements, it will be submitted for airport and local TSA review. After the reviews and any alterations are completed, an approved concept will then be sent out and attached to the ReMAG request. After approval is given the other infrastructure, bid, or permit drawings can proceed.

There are three (3) basic types of checkpoint projects. Whoever initiates the project is the project sponsor.

Airport/Airline-initiated

TSA-initiated

Projects that include Capabilities Acceptance Management Branch (CAMB) gifting or donating Transportation Security Equipment (TSE)

### Airport Initiated Projects

If the project sponsor is the airport owner/operator or the airline initiating checkpoint improvements, the project sponsor may include local representatives of overall airport operations. The project sponsor is responsible for all aspects of the project including basic engineering, hiring of a licensed A&E firm, and submitting concept designs for approval to the TSA design team. Once approved the project sponsor A&E firm will create and submit Issue for Construction (IFC) documents and “as built” records at the completion of the project. The project sponsor will also drive communications, master planning, project management, phasing, and other appropriate design functions, funding and contracting with a systems integrator (SI) and technicians certified to move and install TSE. Examples of airport/airline–driven projects include new or renovated checkpoints and the addition of new lanes driven by increased passenger enplanements. The airport/airline is responsible for all brick-and-mortar work; installation of infrastructure to current TSA standards; movement of any existing TSE that needs to be relocated/moved (phased) for the project, including storage; moves into the final checkpoint location; and providing adequate space for installation of TSE in an approved manner.

On projects where any TSE is relocated or installed, the project sponsor must hire a SI, with technicians certified to move and install TSE, to oversee the movement of the equipment. They are also responsible for any approved Site Acceptance Test (SAT), Operational Readiness Test (ORT), and Integrated Site Acceptance Tests (I-SAT).

If the project is for an additional lane due to growth or capacity increase, the airport/airline is the sponsor. This is a partnership project. The sponsor will perform all tasks described above, and TSA will provide, ship, rig, and install the new additional TSE on that lane. All other lane moves will be completed by the project sponsor.

Project sponsor responsibilities include:

Drawings, ensuring they are approved by the TSA HQ design team, TSA Real Estate Management Division (REMD), and TSA Information Technology (IT) Department

Brick and mortar to meet or exceed TSA CRPG compliance and in accordance with the Airport Security Plan (ASP) and any permits or inspections for the project

Installation, testing, and labeling of electrical and data requirements contained in the CRPG

Movement, installation, and storage of all TSE

If this is a turnkey operation or required due to the nature of phasing, then the project sponsor is responsible for decommissioning and shipping all temporary TSE back to the TSA warehouse

Hiring of SI and technicians certified on the TSE to execute all TSE movement

Hiring, either through the SI or independently through the evaluation team, to assure testing is completed

Additional requirements locally mandated by code, ASP, or other

If Airport leased space is impacted, the project sponsor will work with local TSA and TSA REMD to develop a plan for the TSE movement or phasing into the new area

IT infrastructure, including cabinets and adequate IT Rooms

If the airport/airline driven project is driven by capacity (additional lanes are needed), then this creates an informal partnership. TSA will procure and supply the TSE needed for the lanes; the airport will be responsible for all other items listed above for the lanes’ installation.

(This list is not all-inclusive; all portions of airport/airlines projects are the responsibility of the airport/airline).

### TSA-Initiated Projects

TSA initiated projects include deployment of new or emerging technologies and reduction of equipment due to reduced enplanements or efficiencies gained. TSA is the project sponsor for these projects. Some of these responsibilities are also managed by TSA on airport projects.

Responsibilities of TSA HQ include:

Approving the number of lanes required or permitted at a checkpoint based on airport category, unique security requirements, and passenger volume (once the project sponsor forecasts demand), as well as planning for growth. Typically, this is done with local TSA and TSA HQ staff (working with the project sponsor on airport-initiated projects) to ensure a capacity analysis is completed through the ReMAG process.

Providing regular correspondence of lessons learned and regularly updating stakeholders regarding design and process changes

Performing technical and operational review of designs including coordinating with local TSA staff during design to ensure relevance and operational feasibility

Reviewing the impact of screening protocol changes

Considering local operational and policy issues

Determining the specific equipment types to be used; in some cases, the vendor is unknown due to time suspense and is subject to change when planning for future considerations

Scheduling deployment of new equipment

If an anomalous situation arises, TSA and/or the SI consulting with TSA Occupational Safety, Health, and Environment (OSHE) specialists and/or locally assigned organization OSHE specialists will verify Life Safety Code compliance and coordinate Baseline Hazard Assessments of newly constructed space at handover (if applicable).

Ensuring optimal design to create an environment for a good passenger experience and an area where Transportation Security Officers (TSOs) can perform their security mission unimpeded

Advising on future space planning considerations including projected upcoming technologies, infrastructure, and TSE

Consulting with contractors as required for checkpoint infrastructure improvements for project-sponsored checkpoint projects

For TSA-funded new equipment only, coordinating shipping, storing, transporting, and installing the equipment at the checkpoint

Assisting local TSA staff with the TSA REMD and TSA IT to validate drawings and contract/IT plans

### Capabilities Acceptance Management Branch

The Capabilities Acceptance Management Branch (CAMB) facilitates donation of security screening equipment listed in the Acceptable Capabilities List (ACL) to TSA through a defined process. As TSA qualifies new systems, they will be added to the ACL for donation consideration. TSA will only accept equipment that is on the ACL and approved through a TSA capacity analysis. The process for donation/gifting takes several months to complete and to get to an approval early involvement is key. In a CAMB project, the airport/airline is the project sponsor, and immediate and constant communication with TSA is necessary to ensure all requirements of the project sponsor defined.

The process includes five steps across three phases: Evaluation, Preparation, and Implementation.

#### Phase 1: Evaluation

Step 1: Initial Request – The donor point of contact (POC) should contact their Local TSA POC to indicate interest in donating a Capability and provide all requested information to initiate a Capability Request. Local TSA submits a ReMAG with a Capability Donation Request letter from the donor.

Step 2: Business Case Determination – TSA evaluates the Business Case and Financial Analysis factors to determine whether it is in the best interest of TSA to accept the Capability Donation Request. No additional information is needed from the donor for this step.

Step 3: Memorandum of Understanding – TSA and the donor formalize their intent to move forward with the Capability Donation Request defining and documenting the baseline expectations for both parties.

#### Phase 2: Preparation

Step 4: Support of Donor Responsibilities – Assignment and coordination between the donor and TSA of all necessary pre-implementation activities (e.g., design, procurement, site prep, delivery, installation, acceptance testing) occurs and is documented in the Terms and Conditions for Acceptable Capability.

#### Phase 3: Implementation

Step 5: Capability Offer and Acceptance – Formalizes the transfer and acceptance of Capability from the Donor to TSA via an Offer Letter and TSA Acceptance Letter

### What Criteria Does TSA Use to Assess Capability Donation Requests?

A Capability Donation Request is evaluated on Business Case and Financial Analysis factors to provide a comprehensive Business Case Determination. The factors include the following, but may be tailored depending on the Capability Request:

- Technology: Is the technology on the ACL?

- Demonstrated Need: Does TSA’s capacity analysis validate the request?

- Staffing: Can TSA staff the donated TSE?

- Training: Does the donation require additional training?

- Operational Impact: Will impacts to operations occur, such as an increase to wait time?

- Project Impact: Will there be positive or negative impacts to other TSA projects, such as deployments of other TSE?

- Site Readiness: Can the local airport accommodate the donation without infrastructure changes?

- Deployment: Can the Capability be deployed with minimal disruption to TSA (resourcing, SI costs, etc.)?

- Sustainability: Does the Capability increase immediate costs of training, staffing, and/or maintenance at this airport?

- Financial Analysis: Based on the lifecycle cost of the donation, does the benefit of accepting the donation outweigh the cost to TSA?

- Other benefits or risks

Please see the following link to the CAMB site where more information on the Capability Acceptance Process (CAP) is housed: https://www.tsa.gov/for-industry/capability-acceptance-process. Interested donors can also send an email to CAP@tsa.dhs.gov.

| Financial Roles and Responsibilities | Overview and matrix of financial roles and responsibilities by role during project planning |  |
| --- | --- | --- |

## 2-2	Project Planning and Design

The intention of security screening is to detect, deter and prevent criminal activity, to including terroristic events. This includes the transport of explosive, incendiary, or other dangerous/prohibited items aboard commercial aircraft. Sterile areas are defined as those areas where aircraft access is possible and a clear line between screening locations and public is made. Only persons that have undergone approved appropriate security/other screening have access. Non-sterile areas are accessible to the public and are located prior to any checkpoint or direct access control point. The SSCPs are critical avenues of circulation from non-sterile to sterile areas for passengers, airport/airline employees, and other items required for commerce/other activities in the sterile area. SSCPs play a vital role in the aviation industry and are an important element to consider in an airport’s overall terminal design. Creating an optimal checkpoint layout, including smooth movement of passengers is paramount to a passengers’ experiences, and meeting the airlines expectations and is critical to accomplishing TSA’s mission.

### Project Design Process and Planning Milestones

The process for designing a new checkpoint generally includes an initial concept design, TSA and airport authority review, final design, construction/installation, and record drawing completion. TSA designers must review and approve all designs. Any deviations from an approved design or design standards outlined in this guide must be approved. Any design or infrastructure work completed prior to an approved design is done at risk.

All new checkpoints shall be on level floors. Newer TSA equipment only allows nominal adjustment to the legs on a level floor

Sufficient square footage to support current TSA technology and screening processes: consideration should be given to making space available in a sensible manner for planned volume growth and associated screening capacity at the checkpoint over the expected life of the checkpoint or terminal, as well as operational flexibility in response to changes in passenger loads, equipment, technology, processes, and security levels.

Ability to secure the non-operating security lanes during SSCP operational hours and the ability to secure the checkpoint after operational hours: may include locking doors, gates, and barriers; automatic exits in accordance with the approved ASP; sufficient passenger queuing space based on current and planned future needs; and/or continuous monitoring by law enforcement/security personnel.

Wheelchair accessibility and allowances for persons with disabilities and/or assistive devices

Minimal interruption or delay to the flow of passengers and others being screened

Effective and secure handling of goods transported from the non-sterile area to the sterile area

Equipment maintenance clearances

Efficient and effective use of terminal space

Acceptable and comfortable environmental factors, such as air temperature, humidity, air quality, glare, lighting, and noise

Safe and ergonomic design

Coordination of power, data, fiber optics, closed-circuit television (CCTV), and lighting at the SSCP

While not required, contingency plans for emergency power during outages and system challenges to ensure the SSCP remains operational

Allowance for non-leased TSA support space as near as practical to the SSCP (including private screening, remote screening, and IT/CPSS server rooms)

Staffing efficiency for TSA and other security personnel

When required, coordination with the Environmental Planning Program Manager (EPPM) to ensure compliance with the National Environmental Policy Act of 1969 (NEPA) and other applicable environmental regulations

Completion of a blast mitigation study for any checkpoint constructed over a roadway or where the front of the checkpoint is near a roadway to ensure appropriate steps are taken to ensure the public and Law Enforcement Officers (LEOs) are protected from threat

### Variance or Deviation from Design

The CRPG provides guidelines and requirements for the space and equipment needed by TSOs to conduct screening and security operations as mandated by TSA. Proper layout and furnishing of the checkpoint are essential to ensuring that TSOs can perform their duties effectively, efficiently, and safely. Careful consideration of the operational controls, environmental and ergonomic configuration, and equipment specified for the checkpoint must be given. Although each airport is different and available space may differ, all checkpoints need to follow the same layout concept. This standardizes training and improves personnel utilization across multiple checkpoints within an airport. Any deviation from this guide must be discussed during the design process and approved by the TSA HQ design team.

During the concept design phase, the A&E designer determines the modification scope of the checkpoint including additional equipment to be deployed and/or relocation of existing equipment. Once a new concept design is created, the existing condition design (demolition) and proposed design (concepts) must be submitted to the TSA design team and the project sponsor for review. TSA design will drive the alterations required reach compliance. Once compliant, the TSA design team will send the concept design to local TSA staff for review and approval of requested changes. Local TSA staff will share the concept design with the airport for comment. TSA designers are available to collaborate via on-site workshops or internet-based meetings to work with the project sponsor and ensure that TSA requirements are met. It is important to note that there may be multiple iterations of the concept design, and version controls should be implemented to ensure accuracy and proper coordination.

Once all changes resulting from the concept design phase have been incorporated, 100% designs of the existing and proposed checkpoint area and leased space, including electrical/data layout, are submitted to TSA for final review in PDF and AutoCAD.

The Deliverable Milestones box below lists the major checkpoint design deliverables that should be submitted to TSA via formal documentation. The Final Design Package box lists the key documents to be issued for approval prior to construction.

| Deliverable Milestones - Requirements Management Advisory Group (ReMAG) Submission - Dimensioned Concept Design Submission (including electrical and data sheets in separate PDF and AutoCAD and all leased and non-leased space) 2. Dimensioned Final Design Submission 3. Site Readiness/Verification Visit 4. As-Built Drawings | Final Design Package - Equipment description sheet - Equipment delivery paths - Electrical/data details/layouts/modifications - Structural load capacity analysis/seismic details (Structural Engineer) - All requirements specified by the Airport Authority - Phasing plan to minimize operational disruption - Glare study, if applicable (Architect of Record [AOR]) - Rigging path - Vibration study, if applicable (AOR) |
| --- | --- |

The TSA Design Approval Process, described below, ensures the requirements of this guide are met while still allowing flexibility to accommodate the unique physical and operational aspects in the field and best plan for the future.

### TSA Design Approval Process

When a new checkpoint layout is required, TSA requires the project sponsor’s A&E designer to begin the process using the criteria described in this guide. These criteria are based on the deployment of new equipment and/or relocation of existing equipment and are typically outlined in a deployment schedule produced by TSA. Equipment layout and dimensional requirements provide quality assurance/quality control that the layout meets general guidelines for a security checkpoint.

All designs are currently to be produced in PDF and AutoCAD (aCAD) 2015 format for compatibility. If the project sponsor wants to model the design in Revit or another program, it is expected that the checkpoint drawing will be created in aCAD using TSA-approved blocks and once approved, converted to Revit. The AutoCAD files should contain a background floor plan of one or more TSA SSCP for the airport. TSA provides dynamic aCAD blocks of their equipment to be used for creating checkpoint designs. All TSA equipment is represented by dynamic aCAD blocks that are standard for all checkpoint designs and should be obtained from TSA. The standard equipment dynamic blocks allow the A&E designer to manipulate and locate the TSA equipment within the checkpoint while allowing only the available configurations as specified by the manufacturer for each piece of equipment. DO NOT MIRROR the equipment blocks. Mirroring the equipment blocks changes the configuration of the block to a non-compliant form. While there are other formats to build and design checkpoints, the checkpoint must be created and submitted in AutoCAD and PDF. When the A&E designer completes an initial checkpoint and layout, existing and final conditions are plotted to PDF and computer-aided design (CAD), and the initial concept design is submitted by email to TSA for review and approval/rejection. TSA reviews the layout and approves or rejects the planned checkpoint configurations. If the concept design is not approved, a working session is scheduled/coordinated by the A&E designer or TSA to modify the initial concept design. The A&E designer manages the conference while taking input from all stakeholders and modifies the layout in real time to ensure the recommendations are fulfilled. Then the A&E designer resubmits the concept design to TSA for approval/rejection. Any work completed prior to an approved concept is done at risk.

Upon approval of the revised concept design, the A&E designer completes the final drawings, which include equipment delivery paths, phasing, layout modifications, and a schedule for provided and relocated equipment. The final drawings should include electrical designs for installing new or modifying existing electrical/data devices and be submitted to TSA for review prior to construction.

The A&E firm should coordinate with the local Airport Authority for approval and/or permitting through the authority having jurisdiction for construction. Depending on site location and project complexity, permits and approvals may or may not be required. This should be determined early in the project. Some sites require Airport Authority approval only. Permitting or approvals may require additional information to be provided in the deliverable, which is beyond the information given in this guide. During checkpoint construction, changes may occur that are initiated by local TSA, on-site conflicts, Airport Authority requests, or other instances unforeseen in design. The A&E designer should coordinate with the TSA designer regarding any alterations to design. Direction for design change cannot be approved by others. This is important to ensure the direction given is not contrary to standards. The TSA designer will create a resolution of any conflicts.

After completing checkpoint installation, the A&E designer will create Record Drawings from contractor’s red-line drawings and post-construction photographs. Following completion of the Record Drawings, the A&E designer will provide both a PDF and AutoCAD 2015 or later) file to the TSA designer and airline/Airport Authority.

### Design Approval Considerations

To achieve approval during the design planning phase, checkpoint designs must fulfill certain criteria as prescribed by this guide, and the attachments and appendices. When designing or reconfiguring a terminal and/or checkpoint, the following aspects are considered by TSA for approval:

Square footage to support TSE, including future growth planning and equipment. When feasible, airports should plan for increased passenger growth and associated capacity planning for the expected lifespan of the checkpoint/terminal. Reach out to the TSA Checkpoint Design Team/Capacity Modeling Team for current rates and standards.

Ensure the correct OEM’s TSE assigned to the airport is being used in the drawings, as well as the equipment bump-out configuration (if applicable)

Flexibility for screening operations and modifications to future processes and technology

Square footage to support passenger queue and other screening functions

Square footage to support TSA leased and non-leased space (when possible, break rooms and Remote Screening Rooms as close as practical to the checkpoint)

Architectural Barriers Act (ABA)/Americans with Disabilities Act (ADA) compliance

Protection of the checkpoint TSE, other equipment, and access control while the checkpoint is in operation and when lanes (mod sets) are closed

Flexibility in response to changes in passenger and security levels

Efficient use of checkpoint space

Coordination of power and data requirements (each lane impacted has current requirements)

Safe and ergonomic design with human factors is taken into account

Ensure employee workspaces are adequate and compliant

Equipment maintenance zones are considered

Perimeter wall of the checkpoint meets minimum requirement of 10 feet or ASP requirement

Lighting meets requirements (minimum luminance level of 30-foot candles[fc]; 60 fc recommended)

Heating, ventilation, and air conditioning (HVAC) study to ensure proper cooling and heating for the SSCP. A full checkpoint is harder to regulate than an empty one.

Any open floor areas reserved for future growth or lanes not approved/other need to be closed off

Any new or re-federalized airport will need to meet the current standards for equipment and electrical/data

### Space Planning

Per the Aviation and Transportation Security Act (ATSA), airports are required to provide TSA the space necessary for an FSD to perform the screening function at every airport in the United States (and its possessions and territories) with scheduled commercial airline service. When designing a new checkpoint, it is important to account for both leased and non-leased space required for TSA to accomplish its mission. Non-leased space includes but is not limited to the SSCP, Private Screening Room (PSR), IT Room, supervisor podiums, Remote Screening Room (may have been referred to as Image Operator or IO Room in the past), temporary hazmat storage and some storage rooms. Leased spaces include, but are not limited to, the TSO training room, TSO break room, and offices not associated with screening functions.

It is imperative to understand if support space is not located at or close to the checkpoint, security lanes may be open for a shorter period to account for the distance the officer(s) must travel.

The TSA Real Estate Management Division is responsible for providing facilities services for all workspace locations by managing office and airport space allocations; lease acquisitions; design, construction, and renovation of tenant space; furniture acquisition, and facilities support. REMD should be engaged early in the planning of new airport construction projects to provide the opportunity to procure quality spaces for TSA. REMD will develop a program of requirements for space, specifications for construction and provide those documents to the airport or lessor’s architect in advance. TSA IT and/or Physical Security may also need to review the space planning considerations. Local TSA will coordinate with the TSA security components as necessary. More information can be found in the sidebar (right).

| TSA Space Guidelines | Outlines the planning methods used by the Office of Finance and Administration to assign and allocate TSA space; designed to assist in making efficient and practical decisions about acquiring and planning TSA workspaces |  |
| --- | --- | --- |

### Lighting and Glare Considerations

Lighting requirements for a new checkpoint must meet local and/or national codes and the luminance levels identified in American National Standards Institute/Illuminating Engineering Society of North America (ANSI)/Illuminating Engineering Society of North America (IESNA)) RP-104. In locations where critical decisions are made based on visual evaluation (e.g., at bag inspection tables or the Travel Document Checker [TDC] podium), a minimum luminance level of 30-foot candles (fc) is required (60 fc are preferred). Emergency lighting, as required by the building code, should also be a part of the overall design for SSCPs. If the lighting at the checkpoint impacts sight abilities, screening may have to be suspended.

TSA does not provide overhead lighting. The airport is required to provide sufficient overhead lighting to support the screening functions at the checkpoint.

In addition to sufficient lighting, glare must be considered. Reflectance adjacent to task-critical displays must be mitigated. Surfaces adjacent to displays must have a dull matte finish. Natural overhead light (i.e., windows, glass ceilings, glass walls) can cause or exacerbate glare issues and must be addressed. Reference Military Standard (MIL-STD) 1472g regarding filter control of light sources to mitigate potential glare issues.

All lighting designs must be reviewed by the work group to identify any issues with window or monitor glare, and to ensure there is adequate lighting at the search table and TDC locations.

### National Environmental Policy Act

NEPA requires all federal agencies to consider the environmental impacts of proposed actions before decisions are made and actions are taken. An evaluation process, called an environmental review, determines whether the proposed project may have significant impact on the environment. NEPA reviews are best performed alongside the project planning process to reduce the potential for delays or added cost.

NEPA reviews are initiated by the TSA Project Manager or Deployment Coordinator by submitting a secure Proposed Action Questionnaire to the EPPM or by contacting TSAEnvironmentalPlan@tsa.dhs.gov.

| NEPA Fact Sheet | Overview of NEPA applicability, implementation, and benefits |  |
| --- | --- | --- |

### National Safe Skies Alliance, Program for Applied Research in Airport Security

As noted on the Safe Skies website, the Program for Applied Research in Airport Security (PARAS) is an industry-driven, applied research program that develops near-term practical solutions to security problems faced by airport operators. PARAS is managed by Safe Skies, funded by the Federal Aviation Administration (FAA), and modeled after the Airport Cooperative Research Program of the Transportation Research Board. Projects are developed from Problem Statements submitted by industry professionals. Completed findings are available on the Safe Skies website. All findings have had extensive participation in their development (including contributions of content) by federal agencies, industry trade associations, and individual architects, engineers, security consultants, and other subject matter experts (SMEs).

Available on the Safe Skies website (https://www.sskies.org/paras/reports/) is PARAS Report 0028, Recommended Security Guidelines for Airport Planning, Design, and Construction. The guidelines are not government regulations or requirements; they are a compendium of real-world experience and best practices developed by outstanding professionals in the field.

### Media Inquiries

New and updated security equipment is often a topic of interest for local media. If your airport receives a media inquiry about new security screening equipment installation, contact the Office of Public Affairs at TSAMedia@tsa.dhs.gov.

# 3	Security Screening Checkpoints

SSCPs consist of TSE and non-powered ancillary equipment. The equipment in this guide is listed in the order that a passenger encounters it, from the non-sterile area to the sterile area.

Every checkpoint has essentially the same elements, which are site-adapted to the existing conditions. While the queue and composure areas may vary significantly from checkpoint to checkpoint, the screening lanes are fairly consistent with the type of equipment deployed, even though the equipment footprint can vary by manufacturer.

| Revision | Date | Description of Changes |
| --- | --- | --- |
| 0 | 12/17/2018 | Initial Publication |
| 1 | 01/06/2020 | Attachments Update; SSCP Boundaries Update |
| 2 | 09/30/2021 | Updated Format; Updated FF&E Guide; Revisions to 3-1 and -3-3; Updated required opening under Securable Access to 32 feet-9 inches; Added Exit Lane Toolbox to 3-5 |
| 3 | 09/30/2022 | Updated FF&E Guide in 3-2; Updated Exit Lane E-Mail and Pamphlet in 3-5; New Checkpoint Branding Section in 3-8 |
| 4 | 08/01/2023 | Updated FF&E Guide in 3-2; Updated Exit Lane Pamphlet in 3-5; Added new section on TSA One Stop Security Pilot Program in 3-7; Updated embedded Checkpoint Branding information; Updated minimum barrier height from 5 feet to 4 feet to reflect what is available in the FF&E guide |

## 3-1	SSCP Elements

SSCPs are created by combining standard one- and two-lane module sets (or mod sets). A typical one-lane module set generally consists of:

- Travel Document Checker (TDC) which may consist of the TDC Podium, Credential Authentication Technology (CAT) with or without CAT upgrade kits

- Advanced Technology (AT) x-ray or Computed Tomography (CT) and components (may include Remote Screening Room) Note: Checkpoint Property Screening Systems (CPSS) are the current generation of CT

- Walk-Through Metal Detector (WTMD) and/or Advanced Imaging Technology (AIT)

- Explosive Trace Detection (ETD)

- Bottled Liquids Scanner (BLS)

- Passenger Inspection

- Bag Inspection

- Other security technologies

- Private Screening Room (shared across multiple mod sets)

A two-lane mod set is the same as a one-lane mod set but with the addition of a lane with an AT or CT and associated screening equipment. The module sets are created based on the recommended TSA spacing for passenger ingress/egress, clearance for maintenance activities, and prevention of passenger breaches. Separation of sterile and non-sterile areas provides a controlled and contained screening environment.

A modular design enables TSA to determine the depth and width needed for a set number of lanes. The number of lanes is based on the passenger load and the physical space provided by the airport. Local TSA will need to submit a ReMAG for capacity analysis to TSA Headquarters to assist with determining the number of lanes needed to meet the passenger load in the space allotted for the SSCP. As the number of enplanements per year increases and the equipment and technology evolve, the SSCPs need flexibility to change and expand. Allowances for modifications must be included in the Airport Master Plan.

Vulnerabilities specific to a particular airport will dictate where the checkpoint is situated within the terminal. During periods of elevated threat or special events, temporary SSCPs may need to be installed. If this is a potential option, floor space and temporary utilities should be planned into the terminal design by the airport.

## 3-2	Transportation Security Equipment

The TSE in this guide includes the following:

- Certified Equipment is TSE that has passed testing (i.e., meets certain performance parameters such as detection and false alarm rates) at the Transportation Security Laboratory (TSL) and is ready for operational test and evaluation in the field. This equipment cannot be procured for use in security screening. See the TSA Certified Transportation Security Equipment section for information on equipment that is currently certified.

- Qualified Equipment is TSE that has passed TSA operational test and evaluation and is determined suitable for use by TSA in the field. This equipment can be procured for use in security screening subject to TSA oversight (i.e., on the Acceptable Capability List [ACL] available here: https://www.tsa.gov/for-industry/capability-acceptance-process)

The project sponsor is not required to use the furniture, fixtures, and equipment provided by TSA. The project sponsor may purchase their own equipment provided it complies with TSA design standards, is similar in size and dimension to meet human factor requirements and does not interfere with the TSE operation.

| Checkpoint Furniture, Fixtures, and Equipment Guide | Overview of non-TSE checkpoint FF&E available to airports and TSA sites, including shipment lead times |  |
| --- | --- | --- |

Planning for future technologies allows flexibility and cost-savings but there is also risk—certified technologies do not all become qualified TSE, and specifications may change throughout the development and testing processes. Communicating regularly with TSA designers will ensure potential issues are identified and mitigated in a timely manner.

## 3-3	Planning Considerations for Emerging Capabilities

SSCP technology is a dynamic and rapidly changing field. No matter how carefully an airport is designed to take maximum advantage of the current technology, designs must be sufficiently adaptable to meet the changing threats and support emerging technology. Security screening equipment dimensions and/or processes may change, requiring the entire airport security managerial infrastructure to make important decisions regarding modifications, which the designer must then accommodate. The designer’s task will be easier if the original design has anticipated the need for change and has provisions for expansion. The design files provided in Appendix A are sufficiently adaptable to meet emerging threats and support emerging technology. Electrical and data infrastructure should also be flexible, planning for approximately 20-25% additional capacity in conduit and/or walker duct. Planning for adaptable electrical/data devices will best support future changes.

## 3-4	SSCP Boundaries

The boundaries of an SSCP will vary by airport based on SSCP configuration, airport security requirements, and TSA requirements for a particular checkpoint. Typically, the SSCP length starts at the TDC, extends through the checkpoint elements discussed in this section, and ends at the checkpoint exit, which could be at or near the egress seating area or Supervisory Transportation Security Officer (STSO) podium. The SSCP width is the wall-to-wall width of the checkpoint, including all the screening lanes and, where applicable, a co-located exit lane. All walls adjacent to the non-sterile side or running the length of the side of the SSCP need to be at least 10 feet high to prevent the passage of prohibited items from the non-sterile area to the sterile area. Floor-to-ceiling walls are preferred whenever possible. If floor-to-ceiling walls are not possible, the project sponsor should check the Airport Security Plan (ASP) and local TSA regulatory to ensure compliance. They may require mesh or another barrier that goes higher than the standard 10 feet of wall. The 10 feet high or full-height wall should be installed from the entrance of the TDC to the exit of the checkpoint beyond the screening equipment into the sterile area. This wall separates the SSCP from other tenants and activities in the terminal, preventing intrusion into the screening process. Checkpoint boundaries are to be designed according to this document. When an airport is installing a space for future lanes, the area between proposed and future space shall be separated by 10 feet high or full-height temporary wall. In the future, new technology may extend the current boundaries to include additional equipment and functions within the checkpoint or equipment and functions located remotely within the airport. Front walls separating the SSCP from the non-sterile area should be securable when screening operations are not occurring. Where multiple mod sets are installed, separate closures should be used to give the ability to close off access to mod-sets not in use. The preferred location of the front wall is just forward of the divest lanes. Proper TSO egress must be provided if there is inadequate room for an access gate. The secure side of the checkpoint can be open or securable, however, if the walls are not floor-to-ceiling, there should be a closure at the back of the checkpoint to prevent introduction of items over the wall while the checkpoint is closed. Work with local Regulatory to ensure standards are met.

### Securable Access

Securable access to the checkpoint can come in multiple forms. TSA utilizes securable access to prevent unauthorized access during screening operations. If an airport wants to utilize securable access to prevent access during non-TSA operations, the solution must be fully enclosed. If the airport does not provide securable access to prevent access during non-TSA operations, the ASP may require the airport to provide police or other security forces to prevent unauthorized access or introduction of prohibited items. Swing doors are readily available but present issues with moving passenger control equipment to open or close access. Another issue with swing doors is the available opening. Larger checkpoints with multiple TDC positions require an opening of approximately 32 feet, 9 inches per two-lane module set if in plane with the TDC podiums. Swing doors are acceptable for smaller checkpoints where one TDC is required. Sliding doors or grills are also readily available. Sliding doors and grills can be configured to provide larger unobstructed openings. Care should be exercised with sliding devices so that the bottom track is secure to prevent intrusions. Sectional doors are acceptable provided that, when open, the stacked leaves are nonobtrusive. Roll-up doors or grills are desirable because they are secure at the bottom track and can provide large obstruction-free openings. Roll-up or sliding grills should be security type, with anti-pass features such as polycarbonate infill or solid slats.

### Movable Partitions

Movable partitions are ceiling-supported segmented or sectional doors that roll back to a storage area to create a large single opening. Movable partitions have the advantage of an open checkpoint when open and a secured checkpoint when closed. Movable partitions require support from a ceiling-mounted track and, therefore, are limited to those situations where one is available. As with sliding doors and grills, the bottom track of moveable partitions should be secure to prevent intrusions.

### Access Scalability

Scalable access of the lanes in a large checkpoint is important. Checkpoints are staffed based on passenger screening demand. Large checkpoints must have the capability to prevent passengers from accessing unused lanes and circumventing screening. Barriers must be a minimum of 4 feet in height and extend from the first fixed divest point on a lane to a fixed element or hard barrier at the TDC positions. Barriers are to be movable or removable to allow passengers to circulate between staffed lanes. Barriers shall be provided at one per every four lanes of screening.

Access scalability is extremely important. Large checkpoints must have the capability to prevent passengers from accessing unused lanes and circum-venting screening.

## 3-5	Exits from Secure Side

An exit can be co-located with a checkpoint, or it can be located independent of the checkpoint. This lane should be easily identifiable without adversely affecting security. It should also be adequately sized for deplaning passengers exiting the concourse. All local building code egress path requirements must be met.

| Exit Lane Pamphlet | Overview of exit lanes and exit lane toolkit; contact for more information |  |
| --- | --- | --- |

### Framed Security Mesh

A minimum of a 10-foot or full-height wall is required to separate the checkpoint from the exit lane or the sterile area from the non-sterile area. This height impairs the ability for un-cleared passengers to pass prohibited items to a cleared passenger. This requirement should be coordinated with the Airport Authority when a new checkpoint is being considered or an existing checkpoint is being reconfigured and the exit lane needs to be modified. Should the 10-foot wall pose an issue for the HVAC and/or sprinkler systems, use of an eight-foot wall may be acceptable with the addition of a framed security screen or mesh to allow air and water flow, to make up for the difference in height. Check the ASP and with TSA regulatory to ensure the solution meets local requirements.

For manned exit lanes, an exit lane is typically equipped with a table, chair, and podium for a person to monitor the area and deter those attempting to bypass the SSCP from the non-sterile area. The monitor should be located so that traffic attempting to enter the exit lane from the wrong direction can be intercepted. The exit should also include a duress alarm system to covertly signal alarm of any threat to security, as well as a handheld radio. TSA and the airport may share operational responsibility of the exit lane with other parties, such as the airport operator or an airline carrier. These parties contribute to the design of the exit lane and surrounding area to ensure that unauthorized entry does not occur. Exit lanes must be securable when not in operation.

Unique solutions, such as adding revolving doors or turnstiles, CCTV systems, and/or breach alarms, have been deployed to secure exit lanes. These solutions must allow sufficient space to accommodate the equipment as well as passengers with baggage and/or passengers with disabilities. Another solution is using clear glass panels when an exit lane is adjacent to the checkpoint; this helps deter breaches since the exit lane would be highly visible by TSA and airport/airline personnel. These elements can be combined to create an integrated system that utilizes video cameras, video monitors, sensors, and breach alarms concealed within the architectural elements and tied to a centralized system. This would further tighten security around this sensitive area without relying solely on manpower. In new facility planning and design, SSCP exit lanes should be a considerable distance from boarding gates to allow sufficient time to resolve a breach if one should occur. Automated or unmanned exit systems are the responsibility of the airport should they choose to install it. If airports are considering automated or unmanned exit lane technologies, TSA has developed the Exit Lane Toolbox to help airports select and evaluate technologies on their own through a series of web-based, self-guided tools. For access to the toolbox, please reach out to TSA-MPAC-ExitLanes@tsa.dhs.gov.

## 3-6	Known Crewmember®

The KCM program is a joint initiative between Airlines for America (A4A) and the Air Line Pilots Association, International (ALPA). KCM ties airline employee databases together in a seamless way and enables TSOs to positively verify the identity and employment status of crewmembers in real time.

KCM creates a dedicated screening location and alternative screening procedures for crewmembers to positively verify their identity and employee status in real-time. The crewmembers enter established checkpoints at designated checkpoints, exit lanes, or other direct access portals to facilitate the entry into the sterile area of an airport. KCM screening locations should be equipped with a table, chair, podium, duress alarm, and phone. Design considerations for an optimized KCM include:

Number of positions required based on volume

Electrical and data line requirements for each terminal

Optimized access control points for KCM and other groups

CCTV considerations

Should consider placing KCM near a screening checkpoint for random/other screening

This identity-verification and employment-confirmation system for airline pilots (some air carriers include flight attendants) enhances checkpoint screening efficiency by removing airline crewmembers from passenger screening lines. KCM lanes must be secured when not in operation.

## 3-7	Federal Inspection Service (FIS) Checkpoints

Except for pre-clearance inbound passengers, arriving international passengers are required to undergo U.S. screening before transferring to a domestic flight because the U.S. screening process has different requirements and provisions than screening processes at non-U.S. origins.

Some airports with international flights may decide to have a dedicated FIS checkpoint specifically for arriving international passengers transferring to domestic flights. This requirement depends upon capacity and the willingness of the airport. The screening requirements for a FIS checkpoint are the same as other checkpoints, but the volume varies based on the frequency of inbound international flights.

### TSA One Stop Security Pilot Program

One Stop Security (OSS) legislation was signed into law on December 23, 2022. H.R. 7776 James M. Inhofe National Defense Authorization Act – Sec 7132 One Stop Pilot Program authorizes TSA, in coordination with the Commissioner of U.S. Customs and Border Protection, to implement OSS pilots at not more than six foreign last point of departure airports for a period not to exceed six years. This pilot program facilitates expediting the flow of arriving international transfer passengers from flights originating at participating foreign airports to continue to their final destinations without security re-screening by TSA upon initial landing in the United States.

Currently, all U.S. bound passengers with connecting flights originating in the U.S. are required to be rescreened by TSA at the U.S. port of entry. One Stop Security (OSS) agreements would remove the existing requirement to conduct transfer screening of the covered passenger/baggage population making connections to subsequent flights. However, these passengers must still undergo immigration controls by CBP at the U.S. port of arrival.

To participate in TSA’s One Stop Security pilot, prospective airports must meet all the following security requirements:

Foreign Airports:

Must deploy TSA comparable security, technology, and screening procedures - security screening must be conducted in accordance with an aviation security screening agreement

Use AIT, Checkpoint CT, and EDS all in primary use; AT2 X-ray systems may be approved only with investment plans to upgrade to CT systems

Use algorithms capable of meeting TSA detection standards

Have ETDs available at all screening locations

Have an aligned Prohibited Items List

Must not allow commingling of passengers not screened at the OSS airport

Must permit continuous on-site assessment of implementation of security measures by TSA Aviation Security Advisor

U.S. Airports:

OSS Passengers and their hold baggage (checked baggage) arriving from participating foreign airports must be segregated upon arrival at the U.S. port of entry. Passengers may not access their checked baggage until the arrival at their final destination.

Upon arrival in the United States, OSS passengers and non-OSS inbound passengers must be segregated. Arriving OSS passengers from participating foreign airports cannot meet other arriving international passengers, those passengers’ property, or other persons who have not been screened or subjected to other appropriate security controls required for entry into the airport’s sterile area.

For additional information on One Stop Security and all pilot program security requirements please contact TSA’s OSS Program Manager Keith Mueller at Keith.Mueller@tsa.dhs.gov

Requirements Summary Table – SSCP Boundaries and Exits from Secure Side

| SSCP Boundaries |  |
| --- | --- |
| Section 3-4 | All walls adjacent to the non-sterile side or running the length of the side of the SSCP need to be at least 10 feet high to prevent the passage of prohibited items from the non-sterile area to the sterile area. Check with regulatory and the ASP for compliance. |
|  | The 10-foot high or full-height wall should be installed from the entrance of the TDC to the exit of the checkpoint beyond the screening equipment into the sterile area. |
|  | When an airport is installing a space for future lanes, the area between proposed and future space shall be separated by a 10-foot high or full-height temporary wall. |
|  | Front walls separating the SSCP from the non-sterile area should be securable when screening operations are not occurring. |
|  | Securable Access |
|  | Proper TSO egress must be provided. |
|  | Swing Doors: Larger checkpoints with multiple TDC positions require an opening of approximately 32 feet, 9 inches per two-lane module set if in plane with the TDC podiums. |
|  | Sliding Doors/Grills: Bottom track must be secured to prevent intrusions |
|  | Sectional Doors: Stacked leaves must be unobtrusive. |
|  | Roll-up Doors or Grills: Roll-ups must be security type with anti-pass features such as polycarbonate infill or solid slats. |
|  | Access Scalability |
|  | Large checkpoints must have the capability to prevent passengers from accessing unused lanes and circumventing screening. |
|  | Barriers must be at least 4 feet in height and extend from the first fixed divest point on a lane to a fixed element or hard barrier at the TDC positions |
|  | Barriers are to be movable or removable to allow passengers to circulate between staffed lanes. |
|  | Barriers shall be provided at one per every four lanes of screening. |

| Exits from Secure Side |  |
| --- | --- |
| Section 3-5 | Exits must be easily identifiable without adversely affecting security. Exits must be adequately sized for deplaning passengers exiting the concourse. Exits must meet all building code egress path requirements |
|  | A minimum 10-foot or full-height wall is required to separate the checkpoint from the exit lane. Note: This requirement should be coordinated with the Airport Authority when a new checkpoint is being discussed or an existing checkpoint is being reconfigured and the exit lane needs to be modified. |
|  | Exit lane should be equipped with a table, chair, and podium for a person to monitor the area, radio and signal duress alarm. Electrical and data may also be required |
|  | Monitors should be located so that traffic attempting to enter the exit lane from the wrong direction can be intercepted. |
|  | All solutions must allow sufficient space to accommodate the equipment as well as passengers with baggage and/or passengers with disabilities and TSO work area egress routes. |
|  | For new facility planning/ design, exit lanes should be a considerable distance from boarding gates to allow sufficient time to resolve a breach and relieve unnecessary congestion from the area. |

## 3-8	Checkpoint Branding

Presenting a strong, unified brand identity for TSA is essential to earning the respect and trust of the traveling public. TSA brand identity should be reflected in every touchpoint a passenger has with the agency and fully incorporated as much as possible to promote a consistent and optimal experience for customers. Security Screening Checkpoints (SSCP) represent the public’s most frequent and important interaction with the TSA brand and is TSA’s greatest opportunity to reinforce brand identity.

| Checkpoint Branding Identity Concept | Branding and advertising layout examples for consideration. Catalog of branding and advertising assets available for use. Equipment branding examples. |  |
| --- | --- | --- |

# 4	Checkpoint Approach and Passenger Queue

The checkpoint approach and passenger queue zones play an integral part in passenger experience and a critical role in determining screening time. This section outlines requirements and key considerations for planning the checkpoint approach and passenger queue.

| Revision | Date | Description of Changes |
| --- | --- | --- |
| 0 | 12/17/2018 | Initial Publication |
| 1 | 01/06/2020 | Queue Management updated; Canine Enhanced Screening (CES) updated |
| 2 | 09/30/2021 | Format Updated; Spec Sheets Removed Where Appropriate; Illustrations Updated; Updated Signage Information; Updated Automated Wait Time to latest Crowd Movement Technology; CES Updated; Updated TDC/CAT Options and Information including Infectious Disease Shielding |
| 3 | 09/30/2022 | Updated Canine Screening Information in Section 4-2; Removed redundant TDC/CAT information in Section 4-3 |
| 4 | 08/01/2023 | Updated TSA signage in 4-1; Added LAS Innovation Checkpoint Queue Example to 4-2; Updated TDC and CAT-1/2 in 4-3; Added information on TSA PreCheck® |

## 4-1	Pre-Screening Preparation Instruction Zone

The Pre-Screening Preparation Instruction Zone begins as early as the curbside ticket counters and typically ends at the TDC. This zone should incorporate architectural features of the airport and be designed to provide a passenger environment with reduced noise, comfortable lighting, adequate spacing, and other calming features. Signage, instructional videos, and “ambassador” staff or volunteers, when available, should be used to properly prepare passengers for screening, enhance ease of movement through the SSCP, and increase efficiency and throughput.

### Passenger Preparation for the Screening Process

Passenger preparation and divest coaching during the queue waiting process can substantially reduce screening time. Video monitors or video walls in the passenger queue can prepare the passengers for the standard screening process if deployed in an effective manner. The videos provided by TSA HQ can be played on a continuous loop or be motion-activated. Viewing distance is a factor with video presentations; video monitors should be close enough that the passengers receive the information presented and spaced so that a passenger is never out of sight of the next monitor. If a suitable location is not available close to the queue, a video wall could present the information. The video wall has the benefit of addressing the entire queue instead of groups of passengers at a time. Video presentations can include interactive avatars to answer specific passenger questions as they approach divest.

### TSA Signage

Space should be allocated for signage when designing a new checkpoint. Electrical considerations for digital signage may be required in the checkpoint design. Simple and effective checkpoint signage approved by TSA HQ Strategic Communications and Public Affairs can direct and instruct passengers on screening requirements and procedures. TSA signs are available as 11-inch x 14-inch or 22-inch x 28-inch frames that can be mounted on top of a floor stanchion. The four categories of signs are TSA mandatory signs, TSA instructional signs, TSA directional signs, and TSA local signs. Refer to the most current version of the TSA Airport Signage Guidelines and information about all TSA-approved airport checkpoint signage, including specific sign descriptions and where to locate these signs within the checkpoint, on the TSA Intranet. For additional information, including the site address and access to the site, please reach out to the Security Operations Airport Signage Team at OSO.communications@tsa.dhs.gov.

| ReadySeeGo (RSG) Program Information Sheet | Overview, layouts and real-world deployment examples of RSG information totems |  |
| --- | --- | --- |

#### TSA Mandatory Signs

TSA mandatory signs display critical information and TSA policies. These signs need to be visible from both sides, prominent, easy to read, and located along the path of departing passengers without obstructing queue lanes or being a safety hazard. These signs should not be clustered together in a way that larger signs block smaller signs or multiple instructions create information overload for the passengers.

#### TSA Instructional Signs

TSA instructional signs provide passengers with instructions on the screening process. These signs advise passengers on how to properly divest their possessions and place those items in the bins. TSA instructional signs can be mounted in the same way as the TSA mandatory signs or displayed on walls near the divest tables.

#### TSA Directional Signs

TSA directional signs instruct passengers where to go during the screening process, including providing direction to separate queue and screening lanes. The goal is to provide clear and concise directions so that passengers react quicker and overall time in the queue is minimized. Directional signs must be elevated so they are easily visible and are not obstructed by passengers standing in line.

#### TSA Local Signs

TSA local signs are designed to meet specific local requirements, such as instructions regarding special equipment, local processing instructions, and any other information deemed necessary by the local FSD. All requests for custom signs to meet local needs or edits to existing national signs should be submitted to OSO.communications@tsa.dhs.gov.

### Crowd Movement Technology

Crowd movement technology actively monitors and reports key performance metrics such as wait times and throughput at checkpoints in the airport. These systems use various protocols to measure passengers entering the checkpoint queue through exiting the screening area. Wait time data can then be broadcast to the passengers, typically through a digital communications device at the entrances of the checkpoint queue, and optionally at the airport/airline’s website. Passengers are then able to make informed decisions on which checkpoint they would prefer to use. The information can also be displayed in the non-secure side of airports, so once check-in is completed, passengers can make their decision in the check-in areas. This technology is currently provided by the project sponsor, not TSA.

| Crowd Movement Technology | Performance and Security Standards |  |
| --- | --- | --- |

TSA has continued efforts towards the FAA Reauthorization Act of 2018 which states that TSA is to report real-time wait times at SSCPs both online and in physical locations at checkpoints. TSA has developed a set of standards that outline the ideal performance and operational requirements of crowd movement technology that TSA would work towards implementing as funding and resources become available. By sharing TSA’s requirements for this technology, we hope to assist airports and airlines in their technology selection while aiming for a seamless integration in the future.

## 4-2	Passenger Queue

The queue is where passengers stand in line at the front of the checkpoint on the non-sterile side. The airport is responsible for all stanchions and hard panel barriers in the queue prior to the TDC (except with canine-enhanced screening [CES]). It is recommended that the queue be bound by solid barrier stanchions on the perimeter and double strap stanchions inside the perimeter to define the queuing lanes from the queue entrances to the TDCs/ CAT. Queue lanes are approximately 3 to 7 feet wide, depending on the queue lane function and the queue space available.

| Stanchions Information Sheet | Overview of stanchion types, pros and cons of each, and graphic examples |  |
| --- | --- | --- |

TSA recommends adding a minimum of 600 square feet in the queue for every checkpoint lane as outlined in the lane layout PDF and CAD files. The queue should be large enough to meet the peak passenger demand without interfering with other functions in the terminal, such as the ticket counter or checked bag processing. Queues should be able to be cordoned off and funneled down to one TDC during off-peak times.

Strap stanchions are inadequate to fully secure the checkpoint. Solid barrier stanchions are required along the boundary of TDC/CAT podium positions and the flanking side limits of the queue must be minimum of 4 feet tall to the checkpoint walls to prevent unauthorized access.

### Queue Management

The queue and its management are the airport’s responsibility but must be able to accommodate TSA programs such as TSA PreCheck® and CES. Appropriate boundaries must be maintained for each type of passenger circulating through the queue or TDC with flexibility and space to accommodate Future Lane Experience (FLEx) programs. This ensures that populations are segregated appropriately to ensure only low risk individuals receive TSA PreCheck® screening, but it is also vital to capacity management of the checkpoint. Increased wait times may result if the passenger queues are not managed correctly. Non-TSA programs that authenticate passengers and/or feed passengers to the checkpoint shall not interfere with the queue and shall be separated from the queue with hard panel stanchions.

The next generation of queue management includes several technologies focused on streamlining passenger queue navigation, screening process preparation, and securing the location.

### Canine Enhanced Screening

Passenger Screening Canine teams must screen individuals during Canine Enhanced Screening (CES) in an open queue configuration (OQC). OQC is a concept screening procedure that provides the requirements and considerations for an expanded Canine Work Area (CWA) at designated checkpoint locations.

    - To conduct CES operations, screening checkpoints must meet and sustain requirements as directed in the Checkpoint and Specialized Screening (CSS) Standard Operating Procedures (SOP).

    - It is recommended that the local Canine Training Instructor (CTI) is involved in the planning processes of establishing the CWA within the queue space. The CWA is the tested and certified area within the queue, prior to the TDCs, where the canine conducts operations. CES operations can only begin once a CTI has certified the CWA for CES operations.

    - The CWA must be positioned inside the queue, with a controlled entry point into the CWA, allowing for the Explosives Detection Canine Team (EDCT) to conduct screening operations prior to the TDC positions.

    - It is recommended that 80% of the queue space exists prior to the passengers entering the CWA.

    - It is recommended that 20% of the queue remains between the TDC locations and the exit of the CWA.

    - The space requirements for a CWA are a minimum of 12 feet wide and a minimum of 35 feet in length.

| CWA 80/20 Example | The CWA under the 80/20 recommendations with the passengers in the same lane as the canines |  |
| --- | --- | --- |
| LAS Innovation Checkpoint Queue Example | Queue Design, Layout, and Product Solutions |  |

### TSA PreCheck®

TSA PreCheck® is a voluntary, expedited security screening program connecting low-risk travelers departing from the United States with smarter security and a better air travel experience. Standard screening requires removal of all items and for those items to be placed on the X-ray belt for screening. With TSA PreCheck®, passengers don’t need to remove shoes, laptops, liquids, belts and light jackets. In June 2023, 88% of TSA PreCheck® passengers waited less than 5 minutes. TSA PreCheck® is currently available at more than 200 airports with 85+ participating airlines nationwide.

Currently, TSA PreCheck® enrollment is conducted by IDEMIA and the official TSA PreCheck® enrollment site is https://tsaenrollmentbyidemia.tsa.dhs.gov/, which may also be reached from https://www.tsa.gov/precheck.

In the coming months Alclear, LLC (doing business as CLEAR) and Telos Identity Management Solutions, LLC will offer TSA PreCheck® enrollment services. Prior to becoming operational, CLEAR and Telos will process test enrollments in select locations. Following this phase, TSA will announce when each enrollment provider is available to all applicants. When these new enrollment providers are fully operational, travelers can enroll through the provider that best suits their needs.

More information may be found at https://www.tsa.gov/precheck. It is important to note that TSA uses unpredictable security measures, both seen and unseen, throughout the airport. All travelers will be screened, and no individual is guaranteed expedited screening.

## 4-3	TDC and CAT-1/2

Prior to entering the security checkpoint for screening of persons and property, each passenger is required to have their identification and boarding pass validated. The document validating function can be supported by either a TDC podium, TDC with removable shelf, CAT-1, or CAT-2. With TDC, security features of the identification and boarding pass are verified manually. The CAT-1 verifies the authenticity of the identification document, and electronically verifies the boarding pass. The CAT-2 adds additional biometric face matching capabilities to the CAT-1. CAT-2 automates the identity verification function that TSA officers normally perform manually, using the same physical credentials that TSA has always accepted. Passenger participation is voluntary and, if a passenger chooses not to participate in a pilot, they can have their identity verified manually instead.

| IDEMIA E-CAT System Datasheet | System Specifications |  |
| --- | --- | --- |

The TDC/CAT-1/2 function is critical to the flow of passengers through the checkpoint, as it can become the bottleneck in the passenger screening process. To prevent this, the queue must be set up properly to direct the passengers into the TDC/CAT-1/2, and the TDC/CAT-1/2 must be set up properly to direct the passengers into the checkpoint lanes.

The following guidelines should be considered when determining placement of the TDC/CAT-1/2:

There shall be space and infrastructure available to accommodate two TDCs per lane which may include CAT-1 or CAT-2 and other future technology considerations.

Space should be available on either side of the TDC/CAT-1/2 for passengers to flow.

Sufficient clearance should be provided between the queue stanchions and the TDC/CAT-1/2 stanchions at a clearance of 8 feet, 12 inches so that passengers can crossflow to a TDC/CAT-1/2 of their choosing.

There should be 6 to 10 feet of space between the back of the TDC officer and the leading edge of the lane to safeguard personally identifiable information.

Reference the latest FF&E Guide for the latest TDC dimensions.

The TDC/CAT-1/2, hard building walls, and solid barrier stanchions should be set up to prevent unauthorized entry and prevent circumvention of the TDC.

Lighting must meet a minimum luminance level of 30 fc, especially at the TDC, AVS and PSR; 60 fc is recommended. Lighting must be evaluated at all checkpoints to ensure requirements are met for existing and newly built checkpoints.

# 5	Passenger and Carry-On Baggage Screening

With more than 650 checkpoints at airports across the country in existence today, equipment arrangements vary based on the approved approach at the time of implementation. Site conditions and local input also impact the layout of a checkpoint. TSA intends for each arrangement to meet baseline standards based on the current threat; however, these standards change often as new capabilities emerge to address future threats at all category airports. This section outlines the requirements and considerations to be made when designing layouts.

| Revision | Date | Description of Changes |
| --- | --- | --- |
| 0 | 12/17/2018 | Initial Publication |
| 1 | 01/06/2020 | AIT OEM Comparison attachments updated |
| 2 | 09/30/2021 | Added AT-CT and CPSS Updates; Revised Attachments and Images in All Sections, Clarified WTMD requirements in 5-8; Updated L3 to Leidos |
| 3 | 09/30/22 | Reorganized section; Moved Labeled SSCP Layout Diagram from Section 3 to Section 5-1; Updated Section 5-5 to include Analogic CPSS base and full information and to remove CPSS components listing and OEM files; Updated Section 5-6 to include Rohde and Schwarz QPS 201 AIT; Added additional CPSS component information; New Section 5-11, Buffers and Decision Points; New Section 5-12, Composure; New Section 5-13, Bins, Rerun Carts and Bin Return; Updated Sections 5-14, Secondary Screening and Section 5-15, Bag Search Table to note security shielding in these locations is now required; Updated illustrations to reflect latest technology and design guidance |
| 4 | 08/01/2023 | Added information for newly qualified CPSS systems (i.e., IDSS mid, Scarabee mid, Scarabee full, Smiths base, Smiths mid, and Smiths full); Minor updates to emphasize importance of planning for CPSS Image Operations/Remote Screening; Updated slope tolerance in 5-6 to include newly qualified CPSS |

## 5-1	SSCP Layouts

As noted in Section 3-1, checkpoints consist of standard one- or two-lane mod sets or combinations of standard mod sets based on a particular arrangement of a given type and quantity of screening equipment that have been previously qualified by TSA. The two-lane mod set is the same as a one-lane mod set with the addition of a lane with AT, CT or CPSS opposite the lane with AT, CT or CPSS and the other equipment (WTMD, AIT) being located between the two lanes. The equipment between the lanes where passengers traverse is known as the “infield”.

| Labeled SSCP Layout | Diagram showing infield, mod set, operator space, etc. |  |
| --- | --- | --- |

On the opposite side of the lane, where the TSOs work, the area is known as the operator space. A two-lane mod set or a combination of two-lane mod sets is the best approach for configuring a checkpoint because it efficiently utilizes screening equipment and TSA personnel. However, a single lane mod-set should be used if the peak passenger load only supports one lane, the checkpoint has an odd number of lanes, or there is an obstruction (such as a column, electrical closet, or chase) that prevents adding a two-lane mod set.

All designs for new checkpoints or reconfigured existing checkpoints should be based on the mod sets of an arrangement prescribed by TSA HQ checkpoint design standards. A graphic representation of the arrangement is presented in the Example SSCP Layout – Standard figure below. Generic ATs, CTs and AIT units were used in the module sets and arrangements, but any manufacturer included in this guide can be applied using the same recommended spacing. Some adjustments to the layout may be required to account for varying equipment dimensions.

The Designer of Record must evaluate the structural floor prior to placement of the TSE, as the live load each floor system can support varies. Section 8 provides additional information on this topic. A&E firms and/or SIs are reminded to consult applicable codes within the airport’s jurisdiction to determine the applicability of seismic restraints for all brands. OEMs have brackets available for purchase.

### SSCP Arrangement

The following figures depict a transition from current and to possible future SSCP arrangements. The dimensions contained in the CAD and PDF files provided in Appendix C serve as guidelines to lay out a checkpoint. Appendix C is available here: https://www.tsa.gov/sites/default/files/checkpoint-requirements-and-planning-guide_appendix-c.docx. Every attempt to achieve the dimensions listed should be made when designing a checkpoint. The spacing requirements are the same regardless of the make and model of the screening equipment used.

Example Generic SSCP Layout – Standard

Example Generic SSCP Layout – Current to Future

## 5-2	Advanced Technology

The Advanced Technology (AT) x-ray is the legacy generation of x-ray equipment. The latest deployed carry-on baggage screening equipment is the Checkpoint Property Screening System (CPSS), which will eventually replace all ATs. The prior generation of CT technology is referred to as AT-CT and includes the Smiths HI SCAN 6040 CTiX. TSA classifies the carry-on baggage screening equipment as AT, AT-CT or CPSS. AT represents the deployment of the AT x-ray units, consisting of the Rapiscan 620DV and the Smiths 6040aTiX. AT includes an Alternate Viewing Station (AVS). The AVS is where a TSO can recall the image of an alarmed bag from the AT while performing a bag inspection.

The Rapiscan and Smiths AT units come standard in a RH or LH configuration. The “hand” depends on the bump-out orientation. The bump-out is the side bonnet on the AT x-ray that juts out from the main rectangular shape. When standing on the non-sterile side of the AT looking at the infeed tunnel, the bump-out is on the right side of the AT main body on a RH unit and on the left side of the main body on a LH unit. The RH and LH AT units are not symmetrical. The LH AT is a 180° rotation of the RH AT with the infeed and outfeed components interchanged. The bump-out orientation should be specified prior to manufacture of the unit.

The operator workstation is independent of the bump-out and can be positioned on either side. This is often referred to as LH operator or RH operator, and the orientation is determined the same way as the bump-out. Operator orientation needs to be determined prior to manufacture of the unit.

It is important to identify the orientation of the bump-out and the location of the operator early on so that the AT unit configuration can be planned for as designed. If relocating the equipment, please ensure the proper bump outs have been planned for. The four possible orientations of the Rapiscan and Smiths ATs are as follows:

RH bump, LH operator

RH bump, RH operator

LH bump, LH operator

LH bump, RH operator

The AT units are also unique regarding the composure length. The AT divest and re-composure length standard is 12 feet but could range from 2 to 18 feet (depending on checkpoint conditions).

Field-verify the equipment type, bump-out orientation, and operator location early in the process to avoid costly and time-consuming delays.

If existing equipment cannot be installed in its original configuration, work orders must be submitted for its reconfiguration.

| Rapiscan 620DV AT and AVS | Overview of equipment, quantity requirements, various views (plan, elevation, side), and additional information |  |
| --- | --- | --- |
| Smiths 6040aTiX and AVS | Overview of equipment, quantity requirements, various views (plan, elevation, side), and additional information |  |

## 5-3	Computed Tomography

The current generation of carry-on baggage screening includes CT x-ray (or CT) scanning and peripheral equipment known as Checkpoint Property Screening Systems (CPSS). The prior generation of CT technology is referred to as AT-CT and includes the Smiths HI SCAN 6040 CTiX. CT x-ray provides three-dimensional images of bag contents to assist in identifying prohibited or dangerous items. CPSS scanning can eliminate the need for passengers to divest some items from their bags as the operator can manipulate images to clear congested bags. CPSS systems will require additional infrastructure and higher voltage and amperage service in the lanes.

The CPSS units, unlike AT units, are symmetrical about the long axis; however, the maintenance zone is not symmetrical. The machine is directional-bound, and blocks cannot be mirrored. The operator workstation location must be noted when ordering equipment. The configuration of the operator or “hand” depends on the location of the operator workstation. When standing on the non-sterile side of the CT looking at the infeed tunnel, the operator is on the right side of the CT main body on a RH unit and on the left side of the main body on a LH unit. The operator orientation should be specified prior to manufacture of the unit.

CPSS come in base, mid-size and/or full-size configurations. These systems include a CT and varying OEM and configuration-specific peripheral equipment. The base system has a manual diverter roller (MDR). The mid-size system adds an auto-diverter roller (ADR) and radio frequency (RFID) readers for the bins. The full-size system adds a bin return system and high-threat containment unit. The full-size system is often referred to by the number of divest positions, number of buffers and number of revest positions. For example, the current standard Analogic full-size CPSS is a “3-2-5” because it consists of three dual divestiture stations, two buffers and five re-composure stations. The specific components and parameters of each system configuration are explained further in the sections below. As of August 2023, the Analogic Base, Analogic Mid, Analogic Full, IDSS Mid, IDSS Full, Scarabee Mid, Scarabee Full, Smiths Base, Smiths Mid, and Smiths Full CPSS are qualified and deployable.

When planning base machines to accommodate oversize bags, one base CPSS system is recommended for checkpoints with six to 11 lanes and two base CPSS systems are recommended for checkpoints with 12 or more lanes. These systems are installed to handle oversized items. A base unit may also be designed if and only if a mid or full-sized unit will not fit.

For new checkpoints and airport reconfiguration projects, remote screening rooms should be taken into consideration. Additional information is available in the CPSS Image Operations/Remote Screening Room below.

| AT-CT Smiths Hi-Scan CTiX | Spec Sheet |  |
| --- | --- | --- |
| AT-CT Smiths Hi-Scan CTiX | 3D Illustrative Example |  |
| CPSS Base, Analogic | 3D Illustrative Example |  |
| CPSS Mid, Analogic | 3D Illustrative Example |  |
| CPSS Full, Analogic | 3D Illustrative Example |  |
| CPSS Mid, IDSS | 3D Illustrative Example |  |
| CPSS Full, IDSS | 3D Illustrative Example |  |
| CPSS Mid, Scarabee | 3D Illustrative Example |  |
| CPSS Full, Scarabee | 3D Illustrative Example |  |
| CPSS Base, Smiths | 3D Illustrative Example |  |
| CPSS Mid, Smiths | 3D Illustrative Example |  |
| CPSS Full, Smiths | 3D Illustrative Example |  |

### Slope Tolerance

Slope conditions occur in approximately 10% of all existing checkpoints. Any newly constructed checkpoints must be built on a slope of 2% or less. When installing a CPSS on sloped floors, consideration should be given to how the leveled CT will interact with the adjacent infeed and exit components.

Leveling feet may be used to accommodate sloped floors but have limited adjustment capability. As a result of this limitation, the longer the CPSS lane, the less slope tolerance that can be accommodated as the leveling feet run out of adjustment height. Each vendor has a solution for minor slopes. Please validate prior to any planning or installation by the OEMs.

| Slope Tolerance Detail | OEM-Specific Slope Tolerance Information |  |
| --- | --- | --- |

### CPSS Image Operations/Remote Screening Room

The next generation of CPSS should include enhanced resolution to increase throughput per lane. Cross-lane imaging establishes communications across screening lanes within a single checkpoint. Primary and alternate viewing station operators can share, review, and adjudicate any image from the lanes within a checkpoint providing more effective and efficient flow control, resource use, and throughput. Remote screening enables image review and adjudication for primary screening outside the physical checkpoint area. Remote screening can occur within an airport and may occur at offsite locations in the future.

A dedicated room, outside the physical checkpoint where screening is taking place, is provided for primary viewing station (PVS) operators to analyze and adjudicate screening images. In addition to providing the same benefits as cross-lane imaging, remote screening incorporates social distancing protocols and may improve officer ability to perform image resolution. Currently, during peak operation hours, additional bag vetting via remote screening is expected to have additional staffing requirements to accommodate the higher throughput rates, but assessment of operations is required prior to final determination. Checkpoints of three or more lanes may qualify for remote screening and all new checkpoints shall include this in their plans. Smaller checkpoints in airports with other checkpoints may be included in the shared solution. As part of the ancillary space requirements, the IO/ RR rooms are non-leased space.

A CPSS resolution room is expected to be a built-in room, preferably within 100 feet of the checkpoint and 295 feet of the intermediate distribution frame (IDF) or main distribution frame (MDF). The airport is responsible for building the room, its associated infrastructure, and millwork for the stations. Data cable runs from the TSA IT room for TSA and the CPSS vendor must not exceed industry standard. CPSS resolution rooms located between two checkpoints can serve both checkpoints. Rooms must have adequate HVAC systems to handle the equipment and personnel loads. Lighting must be controllable to reduce light levels during active screening. Additional infrastructure is required to support the necessary equipment. Carpet and acoustical tile ceilings are recommended to minimize sound levels. A separate workstation is required for each TSO, with privacy screens between workstations. Each workstation should have a dual flat screen monitor in a flexible configuration and a work surface with drawer storage below. One supervisor work location in each room should also be planned for.

Example Remote Screening Room layouts are provided in CAD format in Appendix A. (Appendix A is co-located with this CRPG).

## 5-4	Automated Screening Lanes (ASLs)

ASLs are a property handling system integrated into an existing AT x-ray or AT-CT. These are considered legacy systems and TSA does not deploy them nor will TSA accept them as donation. In place of ASLs, TSA has qualified CPSS full systems, which integrates a Computed Tomography (CT) system with automated bin return capability under the CPSS Program.

## 5-5	Walk-Through Metal Detector (WTMD)

The WTMD is used for passenger screening. Only OEM and authorized service providers are certified and authorized by TSA to relocate, recalibrate, service, and relocate the power cord to the opposite leg of the WTMD.

To minimize environmental and equipment interference with the WTMD, the following guidelines should be applied:

Locate a WTMD a minimum of 24 inches away from all electrical fields created by escalators, trains, conveyors, neon fixtures, speakers, transformers, banks of electrical circuit breakers, buried duct banks, conduit, wire, and receptacles, both overhead and beneath the floor. Higher voltages will require additional clearances, and checkpoints should never be located above facility electrical distribution rooms. (With TSA designer permission, this clearance can be reduced to 18 inches under design limitations, however any reduction is at risk)

Minimize interference from metal in surrounding architecture, including floors, floor supports, doors, metallic framing, wall studs, façade systems, and columns.

Avoid locating the WTMD across expansion joints or in an area prone to surface vibrations created by equipment above, below, or immediately adjacent to the checkpoint such as baggage conveyors, subway trains, heavy truck traffic, etc.

With floating or raised floors or in modular buildings, a minimum of a 10-foot x 10-foot pad constructed with 5-inch-thick standard weight concrete will be required under the WTMD to eliminate excessive vibrations. Floor pieces should not abut the WTMD.

When relocating a WTMD, field-verify the receptacle type (twist lock L5-15 or standard straight blade) to ensure the correct power supply and cord are available in the new location.

The 12-foot power cord should be run tight to the barrier on the sterile side adjacent to the WTMD to prevent the cord from being run across passenger egress or TSA work paths where the cord is likely to be a trip hazard or become damaged

They should have separate transformer/rectifier adjacent to the power cord

Silicone and/or bolt the WTMD to the floor per manufacturer’s specifications by TSA-approved technician. If the floor is carpeted, the carpet must be cut out and the unit bolted down.

## 5-6	Advanced Imaging Technology

AIT provides an additional element of passenger screening by being able to detect a broad spectrum of materials concealed on an individual. AIT is the primary device used for standard screening. The Leidos ProVision, whose portal uses millimeter-wave imaging technology similar to the type of waves a cell phone emits to detect items in question, is currently deployed in most locations.

The Rohde & Schwarz (R&S) Quick Personnel Security (QPS) 201 security scanner uses millimeter-wave radio frequency technology to screen passengers. The QPS system requires only a few milliseconds to scan passengers, which can speed checkpoint-screening operations and increase throughput.

Privacy is protected on both systems through use of a generic outline of a person to indicate to operators the location of an alarm.

### Leidos Provision Installation

The Leidos ProVision 2 AIT provides the same imaging technology as the ProVision AIT, but in a smaller unit. Designers are recommended to follow the same checkpoint dimensional requirements for the ProVision so the footprint of all machines is accounted for in the checkpoint. The height and ceiling clearance requirements for the Provision AIT are 9 feet and 9 feet, 3 inches respectively. The height and ceiling clearance requirements for the Provision 2 AIT are 7 feet, 9 inches and 8 feet respectively. When possible, the Touch Control Operator Panel (TCOP) should be positioned on the WTMD side, and the power leg must be on the x-ray side. The TCOP may not be mounted on the control leg.

It is best to provide power for the AIT and corresponding Uninterruptible Power Supply (UPS) from under an adjacent AT/CT/CPSS x-ray lane. The UPS should be located such that it is not a trip hazard to passengers or to the AIT operator at the Touch Control Operator Panel (TCOP). When a barrier is located between the Leidos AIT control leg and the AT/CT/CPSS, the AIT power cord (25-foot) can extend to the receptacle under the infeed or outfeed conveyors of the AT/CT/CPSS along the barrier in an appropriate surface-mounted raceway. In some cases, an ADA gate or a WTMD is located between the Leidos AIT control leg and the AT/CT/CPSS. Extending the AIT power cord across passenger flow is a safety hazard and is not acceptable. For the Leidos AIT only, when all other options have been exhausted, the unit can be rotated 180° to locate the control leg adjacent to a barrier so that the cord can be extended along the barrier in an appropriate surface-mounted raceway. When there is passenger flow on both sides of the AIT, such as an ADA gate and a WTMD, the potential exists for tripping, damage to cords, or unplugging of equipment. In this case, a separate power device should be provided for the AIT and co-located ETD unit. Designers should review and understand the Leidos AIT cord lengths to avoid unnecessarily rotating the AIT. The 16-foot, 0-inch shipped USB cable can be substituted for a 25-foot cable in the field if necessary. The power cable shipped with the unit may be replaced with TSA designer approval.

Designers are reminded to consult local building codes, regulations and guidelines within the airport’s jurisdiction to determine the applicability of seismic restraints for all brands. Manufacturers have brackets available for purchase and use if required by code.

### Rohde and Schwarz QPS Installation

The Rohde & Schwarz (R&S) QPS has several different capabilities that impact design, installation, and use. The height and ceiling clearance requirements for the QPS201 are 7 feet, 6 inches and 8 feet, 6 inches respectively. The open-air design uses two symmetrical panels, with the primary panel shown with a human-like graphic printed on the front of the panel and the secondary panel facing the back of the person being scanned (R&S Fig 1). The floor plate contains wiring between the panels, the power, whether from a mains receptacle or UPS. It can be supplied from either the primary or secondary side panels, thus providing flexibility for power receptacle location and UPS placement. When using a UPS, it should be placed such that it does not create a tripping hazard or otherwise impede the flow of foot traffic.

A R&S certified technician can slide both the primary and secondary panels across the entire width of the floor plate for servicing, eliminating the need for a service and maintenance clearance zone when positioned adjacent to other equipment or barriers. The minimum distance to adjacent equipment is 4 inches or the OEM minimal (whichever is greatest) which enables the QPS systems to be placed in side-by-side configurations and against a wall if required.

The internal distance between the panels is wide enough to comply with ADA requirements for wheelchair pass through. However, at this time, separate ADA access is required.

With the QPS201, up to four additional resolution stations can be added to the lane for additional throughput during high-capacity flows. The resolution stations provide TSA the ability to forward scan results to an area away from the primary screening option to resolve alarms and conduct pat down procedures while continuing to scan additional passengers. The standard TSA operational configuration for the QPS201 includes two resolution stations. The operator control panel receives power directly from the primary panel and all resolution stations can utilize Power over Ethernet (PoE) technology from the QPS, thus eliminating the need for additional power receptacles (see R&S QPS Electrical Design and Physical Design.pdf and R&S QPS Connector Positions.pdf). The operator touchscreen monitor should be electrically connected to the primary panel and positioned so the operator is facing the person being screened.

For design purposes, each QPS201 should have its own dedicated circuit rated for 20amps at 208V with two L6-20 receptacles installed within 25 feet of the installed unit. When using the UPS (recommended by TSA), the design should include a 30amp 208V circuit with a receptacle placed within 10 feet of the UPS installation location. The UPS should be placed within 25 feet of the installed unit.

R&S Figure 1:  Primary panel with human-like image and position of operator control

### Co-located ETD at AIT

ETD units on a movable stand, cart, or table are located at the exit of the AIT to perform additional screening. The ETD units can be on either side of the AIT exit depending on the location of the power/data. The ETD unit should be fed from the same electrical device as the AIT but remain separate from the AIT’s dedicated circuit. Designers are advised to provide power and data for the ETD unit when developing construction drawings to provide maximum installation flexibility for the optional ETD function.

### Slope Tolerance

The Leidos AIT can be installed on an inclining or a declining floor within the maximum manufacturer- recommended slope. Depending upon the slope of the surface the system is installed on, the inner floor of the ProVision system will also be at an angle. These tolerances pertain to the technical functionality of the equipment and do not consider building codes or ADA accessibility. To ensure accommodation of ADA passengers, designers should consult enacted codes in the jurisdiction.

The Leidos ProVision system can be operated as follows:

If the floor slope is parallel to passenger travel, the maximum floor slope is 3.6% for an AIT1 and 2.8% for an AIT2. Any slope greater than the maximum would require the airport/airline to build platforms to include ADA compliant steps or handrails.

If the floor slope is perpendicular to passenger travel, the Leidos AIT cannot be installed unless the unit can be rotated parallel to the slope. This may be possible at checkpoints with unique shapes.

At the maximum 3.6% slope, the internal floor of the Leidos AIT would have a 1:20 slope after adjusting the downhill leveling screws to their maximum extension. In the normal scanning position, this is equivalent to standing with one foot elevated approximately 1 inch relative to the other and is not normally noticeable.

At the upper maximum slopes, compensatory steps such as leveling the machine’s feet and/or adjusting the floor mat position may be necessary. Designers should contact the manufacturer’s representative for more information when installing on a sloped floor.

The R&S QPS system can be operated as follows:

QPS uses an automatic posture recognition algorithm to automatically select the part of the image that represents the scanned person. A small slope perpendicular or parallel to the entrance will not affect system performance as the posture recognition automatically compensates for it. The variation of the slightly different orientation of passenger vs panels because the slope is significantly smaller than the variation of position and orientation of passengers that occur in the field when scanning different persons. While the ground under the floor plate must remain smooth, the QPS has demonstrated to perform over the following conditions:

Slope parallel to the entrance of the QPS: +5%, +10%, -5%, -10%

Slope perpendicular to the entrance of the QPS: +2%, +10%, -2%, -10%

| Leidos Provision AIT | Configuration Detail |  |
| --- | --- | --- |
| Leidos Provision AIT2 | Configuration Detail |  |
| Rohde & Schwarz QPS201 | Specifications |  |
|  | Connector Positions |  |
|  | Layout |  |

## 5-7	Divest Table/Divest Automation

With AT and AT-CT, divest tables are provided for passengers to stage their bins side-by-side so that they can deposit their personal items into the bins. The divest table allows passengers to slide their bins to the infeed of the x-ray. Current checkpoints utilize a variety of table sizes and types; however, for new checkpoints or checkpoints being reconfigured, the divest tables are 30 inches wide and 48, 60, or 72 inches long. Two 72-inch tables abutted to the infeed roller or loading table of the x-ray are the standard divest length, but limited checkpoint space or obstructions may require shorter tables or only one table. Divest tables are stainless steel with height-adjustable legs from 27 inches to 32 inches. Implementation of these tables will increase sequencing efficiency through the checkpoint. Lanes that do not have enough depth for 12 feet of passenger divestiture will have a slower throughput.

Most CPSS base and mid systems utilize conveyors, consisting of a series of rollers that facilitate transportation of divested accessible property to the powered ingress conveyors and through the scanner. Divest length on systems can vary depending on design considerations. Divest length on the base systems can vary from 3 to 18 feet, with 12 feet the standard. Divest length on the mid systems can vary from 3 feet to 18 feet, with 12 feet the standard.

CPSS full systems have automated parallel and/or dual divestiture subsystems that allow multiple passengers to divest their belongings into multiple bins. An automated conveyor then feeds the passenger bins into the tunnel for scanning. Typically, full-size CPSS will consist of three dual divestiture stations per system with the option to add more if needed/space allows under the CAP program only.

Full-size CPSS systems are often referred to by the number of divest, buffers and revest components. For example, the standard Analogic CPSS full is a “3-2-5”. That is three dual divestiture stations, two buffers and five re-composure stations.

## 5-8	Barriers

To prevent passengers and items from passing into the sterile area from the non-sterile area without being screened, barriers must be installed to close all gaps exceeding 6 inches across the front width or façade of the checkpoint. All barriers must be flush with the floor and be at least 48 inches in height above the floor. Barriers must be rigid enough to prevent vibrations that could interfere with the WTMD and must be self-supporting to reduce any potential hazard to passengers and personnel at the checkpoint. Standard TSA barriers are made of transparent material, come in 12-, 24-, 36-, and 48-inch widths and are 48 inches tall.

## 5-9	Americans with Disabilities Act/Access Gate

The ADA gate on the passenger side is part of the line that separates the non-sterile area from the sterile area. The ADA gate allows passengers that cannot otherwise traverse the WTMD or AIT to reach the sterile area. It should be installed with the latch on the sterile side so that the gate opens into the checkpoint (sterile area). The ADA gate is typically used by wheelchair passengers, passengers requiring special assistance, or passengers with pacemakers. These passengers are brought from the queue through the ADA gate and taken immediately to an area for secondary screening.

The access gate on the operator side is also part of the line that separates the non-sterile area from the sterile area. It is used only by TSA staff, as a travel path that is free and clear of passengers, to access the sterile side and return bins from the composure/extension rollers to the divest tables.

The ADA/access gate is approximately 44 inches wide by 48 inches tall with a 36-inch swing gate made of non-metallic, transparent material. As noted, the swing direction of the ADA/access gate must always open towards the sterile side of the checkpoint. The latch side should conform to local code by providing enough space to open the gate around adjacent equipment.

There are many types of ADA/access gates on site. Some have a specific right-hand (RH) or left-hand (LH) swing and some have a swing that can be configured in the field. If reusing an existing ADA/access gate for a checkpoint reconfiguration, it is important to field verify the type and swing of gate so that any new/relocated equipment installations can take the functionality into account.

When an ADA gate cannot be installed due to space constraints, the checkpoint must have ADA access via either operator access gates or other ADA code-compliant doorways with a readily available path to the secondary screening area.

## 5-10	Diverter Roller

The manual diverter roller (MDR) is a non-powered, gravity fed, stand-alone roller located on the operator side of any AT, AT-CT, or CPSS base unit at the alarm bag cutout. The MDR allows the PVS operator to physically separate and correctly route suspected bins for additional screening and cleared items for passenger re-composure. If an MDR does not fit in the operator space due to a column or other obstruction, a 2-foot x 2-foot table may be used at the bag cutout location. Use of this table in lieu of an MDR must be approved by a TSA designer. If the table will not work, the MDR may be removed entirely.

| MDR Info Sheet | Dimensions, various views (isometric, side, elevation), etc. |  |
| --- | --- | --- |

CPSS mid-size and full-size systems have powered transfer rollers known as automated diverter rollers (ADR) built into their systems. ADRs use input from the screening technology and/or the PVS operator to physically separate and correctly route suspected bins for additional screening and cleared items for passenger re-composure.

## 5-11	Buffers/Decision Points

All CPSS systems include buffer or decision points that allow for additional decision time prior to directing scanned baggage to either secondary screening or re-composure. The buffers are the powered conveyors located prior or next to the manual diverter (CPSS base) or auto-diverter subsystems (CPSS mid and CPSS full). On the Analogic systems, the powered conveyor on the exit side of the CT machine, under the tunnel extension is considered a buffer. This is not the case for all CPSS systems. Powered conveyors after the MDR or ADR and behind the security glass are not considered buffers but rather non-accessible powered conveyors. Buffers may be able to be adjusted to accommodate a column or other design obstruction issue but this must be worked with the TSA designer.

As noted previously, full-size CPSS systems are often referred to by the number of divest, buffers and revest components. For example, the standard Analogic CPSS full is a “3-2-5”. That is 3 dual divestiture stations, 2 buffers and 5 re-composure stations.

## 5-12	Composure

TSA’s standard composure (most referred to as re-composure or revest) is 12 feet re-composure length, which can be any combination of extension rollers or exit rollers depending on the manufacturer of the AT or AT-CT.

Most CPSS base and mid systems utilize unpowered conveyors, consisting of a series of rollers that facilitate the transportation of divested accessible property from the powered egress conveyors after scanning. Revest length on base systems can vary from 3 feet to 18 feet, with 12 feet the standard. Divest length on the mid systems can vary from 3 feet to 18 feet, with 12 feet the standard.

CPSS full systems may have automated re-composure subsystems that queue cleared bins, allowing multiple individuals to retrieve their items in the order in which the items are cleared. Re-composure stations will hold the cleared items for parallel re-composure. Typically, full-size CPSS will consist of five re-composure stations per system with the option to add more if needed/space allows.

## 5-13	Bins, Rerun Carts and Bin Return

Bins are the containers provided for the transport of divested items through the entire screening process. Bin carts are wheeled carts used to stack and transport large numbers of bins. Four bin carts are standard with AT, AT-CT, CPSS base and CPSS mid systems. Two bin carts are typically positioned at the front of the screening lane and two are positioned at the end of the screening lane. Instead of bin carts, the CPSS full system utilizes an automated bin return system. The automated bin return system is a subsystem that automatically transports empty bins from the re-composure area back to the divestiture area.

All CPSS systems (base, mid and full) come with one rerun cart. Rerun carts are bin carts used to move bins that contain property from the secondary conveyor or search station to the entrance of the CPSS scanner to conduct a rescan. These are typically placed at the end of the reject line.

## 5-14	Secondary Screening

Secondary screening is additional screening that may be required for passengers and their bags when they alarm primary screening equipment. Secondary screening should be placed 2 feet, 8 inches (as a standard) to 5 feet from the end of the lane to minimize the travel time and distance TSOs must carry bags. Secondary screening is typically located in the “dead” operator space on back-to-back lanes or at the end of the lane for odd-numbered lanes. This area should be clear of exiting passengers.

The current secondary screening area typically consists of an ETD unit, BLS, AVS, RFID reader (on some systems), mobile cabinet, and search table.

Security shielding is required on the search table. If this required security shielding cannot be implemented on the search table to separate the passenger from the Officer doing the search, stanchions must be utilized to the best of the site’s ability to distance the passenger from the Officer while still allowing full view of the search.

CPSS full-size systems utilize automated secondary screening subsystems and high-threat containment units. The secondary screening subsystems automatically stage and hold suspected items until a secondary screening operator can convey the bin to a secondary search area. The high-threat containment unit contains bins with either an obvious threat or a potential Improvised Explosive Device (IED) that must be isolated from other accessible property.

### Explosive Trace Detection and Bottled Liquids Scanner

ETD units are used to test carry-on bags that have alarmed at the AT. These ETD units should be contained within a mobile cabinet but can sometimes be found sitting directly on a search table. The ETD machines require operational, testing, and maintenance supplies to be located within arm’s reach of the working area. If a mobile cabinet is not used, then alternative storage is required. ETD manufacturers and their specifications are listed in the General ETD and BLS Spec Sheet embedded below. The ETD units should be co-located with one search table for a single lane and two search tables for back-to-back lanes. These same ETDs are also located at the exit of AIT units as a method of secondary screening for passengers who alarm the AIT. This is referred to as an AIT with Co-located ETD. Refer to Section 5-6 for additional information.

ETDs are extremely sensitive to environmental conditions such as temperature, humidity, and air quality. ETDs should be clear of fumes and exhaust to prevent malfunctioning. The ETD units also have a high heat output and should be vented if placed in a non-standard TSA storage device.

BLS aid TSA in identifying explosive, flammable, or hazardous substances that have been concealed in a benign container. The containers do not have to be open for the analysis to be performed. Using Raman spectroscopy (laser) and electromagnetic technology, BLS units quickly analyze and identify the chemical compositions of a wide variety of solids and liquids, including explosives that are currently on the classified threat list. Manufacturers and the procurement specifications are listed in the spec/cut sheets below, with the ETDs.

| General ETD and BLS Spec Sheet | Overview of manufacturers and additional information |  |
| --- | --- | --- |
| Ionscan 500DT_Smiths | OEM cut sheet |  |
| Implant Sciences QS-B220 | OEM cut sheet |  |
| CEIA BLS | OEM cut sheet |  |
| RespondeR RCI_Smiths | OEM cut sheet |  |
| RespondeR BLS_Smiths | OEM cut sheet |  |

### Mobile Cabinets

Mobile security cabinets provide a secure and vented storage area for secondary screening equipment. The ETD mobile cabinet, which is common in the field today, encloses the ETD and its associated operational, testing, and maintenance supplies. The ETD-BLS mobile cabinet will enclose the ETD and BLS and house the operational, testing, and maintenance supplies for both systems.

The cabinets have wheels for easy relocation, but the wheels should be locked when the ETD and/or BLS is in operation. Power/data receptacles for the secondary screening area should not be located under the mobile cabinets because the bottom of the cabinet does not provide enough clearance for devices or plugs.

## 5-15	Bag Search Table

Bag search tables are used for target bag searches, ETD swabbing, and BLS testing. Their stainless-steel surface provides a clean, contaminant-free surface. Bag search tables are also equipped with wheels for easy relocation, but the wheels should be locked when a table is in use. The back and side panels offer privacy during bag searches but may be removed when the bag search table is located with an AVS. Existing bag search tables are 30 inches by 48 inches. CPSS bag search tables are custom-built to the CPSS systems. The base and mid systems will come standard with 30-inch by 48-inch tables. The full system will come with 30-inch by 72-inch but may revert to 30-inch by 48-inch if necessary. The AVS on the CPSS search tables are not mirrorable for TSA-purchased systems.

Security shielding is required on the search table. If this required security shielding cannot be implemented on the search table, stanchions must be utilized to the best of the site’s ability to distance the passenger from the Officer while still allowing full view of the search

## 5-16	Passenger Inspection

### Private Screening Rooms

Passenger inspection can occur at the screening lanes, at the secondary screening area, or in a private room at or near the checkpoint. PSRs should be installed by the airport using conventional construction and should meet the general minimum requirements as well as all code requirements. The local TSA are responsible to ensure all PSRs have a mirror available for passengers to use after screening.

The standard size of the PSR is 10-feet by 12-feet with floor to ceiling walls or minimum 8 feet. If full height, HVAC and fire suppression must be considered. The intent is to have code-compliant accessibility and a semi-permanent work surface/table and chair for screening. Proper lighting and HVAC should be addressed with the PSR. Where possible, the PSR shall be adjacent or immediately adjacent to the checkpoint. Because built-in PSRs must meet all local and national code requirements, existing spaces provided by the airport must be upgraded to current standards prior to being assigned as a PSR. Security cameras must not have a view into the PSR and no camera globes should be seen from inside the PSR. The PSR walls must be opaque. The following four options for the project sponsor–built PSRs are allowed under this guide for CAT III or CAT IV airports with space constraints:

- Built-in (exterior lock only required)

- Retractable rigid panel rooms

- Expandable curtain

- Ceiling-mount curtain

## 5-17	Composure Bench

Seating at the checkpoint egress allows passengers to put their shoes and jackets on and consolidate their personal belongings after completing the screening process. Airports may purchase their own composure bench. TSA provides composure benches approximately 7 feet from the AIT screening area. Generally, to have composure benches in the checkpoint infield, an area 13 feet or greater is required. This area is typically out of the main passenger flow.

## 5-18	Supervisor Podium

The Supervisory Transportation Security Officer (STSO) should be positioned at a podium on the secure side of the checkpoint. One STSO podium is required for every four lanes of equipment. In airport/airline projects, it might be more advantageous from an aesthetic and operational perspective for the airport to build out a podium. This can reduce the number of single podiums scattered on a checkpoint as well as enhance the appearance and passenger experience. Ideally, larger checkpoints may have a podium built in by the project sponsor, like the one shown in the examples embedded below, near the checkpoint exit. The STSO should be able to perform administrative duties while viewing and supervising the entire screening operation. The location should have an unobstructed view of the checkpoint.

Supervisor podiums have the highest functionality when they are built into the back of the checkpoint out of the flow of passenger traffic. It is recommended that the finishes of the built-in podiums blend with the terminal and checkpoint construction. An elevated podium floor (18 inches from the level of the checkpoint) is optimal for security. The podium can be built on top of existing construction with lightweight wood construction. It should include a screen wall facing the checkpoint with a built-in computer counter. Power and data requirements must be reviewed for each case and confirmed with local TSA.

| STSO Podium | Dimensions and quantity per lane |  |
| --- | --- | --- |
| STSO Podium Plans | 2 Position Plan | *CAD available in Appendix C |
|  | 3 Position Plan | *CAD available in Appendix C |
|  | 4 Position Plan | *CAD available in Appendix C |
|  | 5 Position Plan | *CAD available in Appendix C |

## 5-19	Equipment Dimensional Criteria

As a guide for designing single-, two-, or three-lane checkpoints and determining equipment placement for multiple lane configurations, TSA provides example checkpoint layouts with different manufacturers’ equipment. Multiple configurations are possible; not all configurations may be depicted. When the architect completes the design, the indicated dimension strings must be maintained. The TSA designer can answer questions and provide additional information. The TSA-provided PDF and CAD design examples are presented in Appendix C (available co- located with this CRPG.).

# 6	Emerging Technology (TSA Certified Transportation Security Equipment)

Designs must be sufficiently adaptable to meet changing threats and support emerging technology. If the original design anticipates the need for expansion, accommodating changes to security screening equipment dimensions and electrical and data infrastructure requirements, it will minimize disruptions, cosmetic repairs, and costs in the future.

This section provides an overview of emerging technologies and TSE that is certified, but not yet qualified, so that these future planning considerations may be made where possible. Certified equipment is TSE that has passed testing (i.e., meets certain performance parameters such as detection and false alarm rates) at the Transportation Security Laboratory (TSL) and is ready for operational test and evaluation in the field. This equipment cannot yet be procured for use in security screening.

Planning for future technologies allows flexibility and cost-savings but there is also risk—certified technologies do not all become qualified TSE, and specifications may change throughout the development and testing processes. Communicating regularly with TSA designers will ensure potential issues are identified and mitigated in a timely manner.

Proposed donations of certified TSA for use under a demonstration can be submitted under the Capability Acceptance Process (CAP). Information on this can be found at the following link: https://www.tsa.gov/for-industry/capability-acceptance-process. It is important that a donor contact TSA and execute the CAP prior to procuring any capability that will be donated to TSA or operated in a screening environment. Strict adherence to the process reduces the risk of procuring a capability that may not be accepted or operable by TSA.

| Revision | Date | Description of Changes |
| --- | --- | --- |
| 0 | 12/17/2018 | Initial Publication |
| 1 | 01/06/2020 | Introduction updated; CT OEM, CT, and ASL AT2 Comparisons updated |
| 2 | 09/30/2021 | Updated for CPSS; Revised Attachments |
| 3 | 09/30/2022 | e-AIT moved into Section 5 |
| 4 | 08/01/2023 | Added information on AutoCAT in 4-1 |

## 6-1	Passenger Authentication

### Travel Document Checker – Credential Authentication Technology (CAT) – 2nd Generation CAT (CAT-2) - AutoCAT

The next generation of passenger identification and authentication will include technologies for reducing staffing demands and increasing passenger throughput. New technology can augment TDCs and CAT-1 with the use of facial recognition technology in the 2nd generation CAT (CAT-2). TSA has been approved to upgrade all CAT-1s to CAT-2. The CAT-2 provides additional biometric face matching capabilities to the CAT-1. The use of facial recognition technology will automatically verify that the passenger is who they claim to be with their identity document. TSA is exploring an automated electronic gate (e-gate) variant of the CAT-2 system known as AutoCAT. AutoCAT includes self-service CAT-2 functionalities with the addition of an e-gate for positive passenger control. This solution has the potential to further automate the identity verification process and reduce physical contact between the TSA officer and passengers. It also reduces the level of staffing required to oversee identity screening operations, thus allowing TSA to re-allocate valuable resources to the other pressing security tasks (i.e., baggage screening, alarm resolution). CAT-2s are currently being tested in multiple airports nationwide while the AutoCAT is still under development with the intention to be tested soon.

# 7	Supplemental Information

This section provides supplemental information for consideration.

| Revision | Date | Description of Changes |
| --- | --- | --- |
| 0 | 12/17/2018 | Initial Publication |
| 1 | 09/302021 | Added Section 8-4, Infectious Disease Controls |
| 2 | 09/30/2022 | Updated Section 8-4 to reflect security shielding around the TDC/CAT and divest is at the discretion of the FSD and OSHE but is mandatory around the search table |
| 3 | 08/01/2023 | Changed from Section 8 to Section 7; Updated Security Shielding in 7-4 to remove references to disease control and added example illustrations of search table security shielding |

## 7-1	Emergency Checkpoints

Unforeseeable circumstances and/or acts of nature may arise in which existing checkpoints cannot be utilized or additional/temporary checkpoints are required. The following list provides layout examples for consideration during these situations. Although these layouts utilize AT machines, a CT may be utilized instead.

| 10x30 Temporary Checkpoint Screening Layout |  |
| --- | --- |
| 20x40 Temporary Checkpoint Screening |  |
| Rapiscan Dual Lane Temporary Emergency Checkpoint |  |
| Rapiscan Single Lane Temporary Emergency Checkpoint |  |
| Smiths Dual Lane Temporary Emergency Checkpoint |  |
| Smiths Single Lane Temporary Emergency Checkpoint |  |
| Temporary Emergency Checkpoint CAD File |  |

## 7-2	Closed-Circuit Television (CCTV) Requirements

While cameras are not mandatory at SSCPs they increase the level of security by deterring theft, reducing claims, providing data for resolving security events, and capturing visual records of suspicious activity. Cameras are particularly helpful at unmanned or closed checkpoints and KCM® locations. The number of cameras required varies depending on the checkpoint size, obstructions within the checkpoint, lighting, and quality of the CCTV system. A sufficient number of cameras should be added to cover each lane, all secondary screening areas, and co-located exit lanes. Camera coverage details will be determined by local TSA and the project sponsor coordinating with the airport or local police. Passenger privacy should be protected by ensuring the cameras do not capture views into PSRs. The camera globes must not be viewable from inside the room. Cameras should be positioned to observe the front view of a person’s face and any other identifying characteristics.

Additional lighting may be required for a CCTV system at the checkpoint. This lighting should be provided by the group that funds and maintains the CCTV system.

Where possible, the CCTV should be designed as an extension of the airport’s existing security system. When CCTV is part of an extended system, the equipment should match the existing hardware to minimize maintenance costs and provide operator familiarity. Storage and retrieval of video footage is determined on a site-by-site basis. Viewing stations on the checkpoint for TSA and Law Enforcement use can help to quickly resolve access control events, customer complaints, and thefts as well as increase the security stance of the airport.

The existing security system should provide a minimum of 30 days of recording. The local TSA and law enforcement should be able to access the system at or near the checkpoint. The security system should have a means to maintain an accurate system time. When a CCTV system exists, it is shared between the airport, law enforcement, and local TSA. Sometimes construction documents by the CCTV owner’s designer or other designated firm are required to indicate the CCTV system scope of work for relocating or adding cameras to support the checkpoint reconfiguration or additional new equipment. These drawings are typically full-sized, and consist of the following:

CCTV and electrical system abbreviations, symbols, and general notes

CCTV camera mounting details

CCTV system demolition, indicating components to remain or be removed; for remaining components, includes a CCTV camera schedule indicating focus, aim, mounting, and applicable remarks for each existing CCTV camera

New CCTV system installation, indicating components to remain or to be provided as new; includes CCTV camera schedule indicating focus, aim, mounting, relocation, disposition, and type for each remaining/new CCTV camera.

These drawings are not typically provided by TSA, but they are part of a checkpoint construction contract. TSA provides the operational requirements from the local FSD, along with enough detail for the system to meet the needs of the program. A typical operational requirement is to “…provide a view of people entering and exiting a walk-through metal detector (WTMD) with enough detail to recognize the person and any object they may be carrying.” Specific questions on the generation of requirements should be directed to the Advanced Surveillance Program at ost_asp_video_surveillance@tsa.dhs.gov.

## 7-3	Safety Requirements

SSCPs must screen passengers and their carry-on baggage without compromising the safety of the passengers or the TSOs conducting the screening. Safety requirements and safety-related considerations must be built into the SSCP design from the beginning and should be treated as integral to the design process. The standard checkpoint layouts in this guide are intended to provide starting points, but safety SMEs should be included in every design phase to provide input on concept plans and/or construction drawing packages.

At all times, all checkpoints must maintain a minimum of two paths of egress to exit the checkpoint. This is achieved by providing egress from the checkpoint via the exit to the airport terminal, as well as egress between AT x-ray/CT and a wall or between two operator sides of the AT x-ray. All egress paths through the non-sterile side of the checkpoint shall be secured by an access gate, as described in this guide.

Safety issues related to equipment or layouts that are likely to arise during SSCP design are discussed within the appropriate sections of this guide; however, this guide is not intended to provide an exhaustive list of such issues. Safety experts from each discipline should review all available sources of information, such as national and local building codes, fire codes, best practices, Technical Notes, Job Aids, Occupational Safety and Health Administration (OSHA)/Occupational Safety, Health, and Environment (OSHE) requirements, and TSO injury data to ensure that each SSCP design incorporates the most current knowledge.

The SSCP equipment, including PSRs, must meet all local code requirements and/or American Society of Heating, Refrigerating and Air Conditioning Engineers (ASHRAE) standards for HVAC. When checkpoints are located outside, the Authority Having Jurisdiction (AHJ) should be consulted.

Indoor air temperature, relative humidity and air flow should be maintained at comfortable levels based on the full occupancy, size, and exposure of the SSCP. Air quality should be monitored at the checkpoint to prevent buildup of carbon dioxide from human respiration and to minimize odors.

The SSCP must meet all local building, mechanical, and OSHE code requirements. New construction or renovated areas with complete HVAC system replacement should achieve minimum ventilation rates and other measures intended to provide acceptable air quality and minimize adverse health effects, as specified in ASHRAE 62.1, Ventilation for Acceptable Indoor Air Quality. The SSCP baseline hazard analysis will evaluate air quality and environmental conditions to identify potential deficiencies.

## 7-4	Security Shielding

To further increase security at the checkpoints, certain engineering controls and processes may be implemented. Physical shielding may be utilized in the TDC/CAT and divest areas at the discretion of the FSD and OSHE. Physical shielding is required in the secondary screening areas. These controls do not eliminate the need for personal protective equipment (PPE). Shielding adds a layer of protection to TSA personnel but should not be taken as a substitution for other controls. Shielding should not inhibit the divestiture or search processes by creating unhealthful ergonomic or lifting hazards. Shielding should also not inhibit the ability to hear between passengers and federal employees or contractors. When applicable, guidance should be provided to passengers to indicate where to stand and/or stop via signage or floor stickers/mats to ensure proper distance measures are observed. Shielding panels may be procured separately or may come as an option with the purchased system.

Air flow should be taken into consideration at all Checkpoints. This is strictly the responsibility of the Airport Authority.

Illustrative Example of Search Table Shielding with Side Panel Opening

Illustrative Example of Search Table Shielding with Center Panel Opening

# 8	Structural Considerations

This section summarizes structural considerations not addressed elsewhere in this guide.

| Revision | Date | Description of Changes |
| --- | --- | --- |
| 0 | 12/17/2018 | Initial Publication |
| 1 | 1/6/2020 | General Requirements updated |
| 2 | 09/30/2021 | CPSS Updates; New Weights and Loads Table |
| 3 | 09/30/2022 | Updated weights and loads table in Section 9-1 for Analogic CPSS |
| 4 | 08/01/2023 | Changed from Section 9 to Section 8; Updated Weights and Loads table to include newly qualified CPSS in 8-1 |

## 8-1	Structural Overview

### General Requirements

The project shall be designed and constructed to support all approved TSA-designated equipment and the building’s dead/live loads and shall be built in accordance with the local building codes and other recognized national standards. The following are items to be considered when designing the support floor:

Raised/access floors are permitted but must meet all design requirements. It is the responsibility of the airport/airline project sponsor to remedy TSE operational issues that arise from the construction of any checkpoint to include mitigation remedies.

A standard weight concrete pad should be placed under the WTMD when a raised/access floor is used. The minimum size of the concrete pad should be 10-feet x 10-feet with 5-inch depth.

Vibrations from floors or equipment below the checkpoint and around equipment should be minimized. Contact the OEMs for further information.

Avoid placement of the WTMD on expansion joints or walker duct plates

Avoid placement of other TSE legs on expansion joints or walker duct plates

All new checkpoints shall be on level floors. Newer TSA equipment only allows for nominal adjustment to the legs on a level floor

Clear rigging paths should be called out early in the drawing to ensure smooth introduction and installation into the checkpoint

Floor protection is to be provided when moving or relocating equipment.

| Weights and Loads Table For Passenger and Carry-On Bag Screening TSE |  |
| --- | --- |

## 8-2	Seismic Anchoring, Fire Codes, and Other Local Codes

The project sponsor is responsible for checking local codes for requirements and compliance, including fire and seismic. TSA does not require seismic bracing/anchoring, but it may be required by local codes. These seismic requirements must be identified and communicated in the early planning stages and plans. Raised or false floors can be particularly problematic when installing seismic bracketing. In these cases, a seismic analysis may be required to secure equipment properly, and the vendor should provide the information needed.

# 9	Equipment Design and Engineering Requirements in the TSA Checkpoint Environment (Future Equipment Vendor Submissions)

Almost all SSCPs are in existing airport facilities across the federalized system. Most of these are in spaces that have been appropriated for screening and were not originally intended for screening passengers and their belongings. As a result, impacts and concessions may be necessary in certain cases to provide the best screening possible in spaces that are limited. This, combined with International Building Code and Airport Authority requirements, makes for a challenging environment for equipment and processes. The following is intended for equipment manufactures to understand the checkpoint environment and its limitations so that new equipment being developed can work within existing TSA checkpoint spaces. TSA procures most of its equipment with the goal of hitting full operational capability (FOC) across its 440+ airports and 2,200+ lanes.

| Revision | Date | Description of Changes |
| --- | --- | --- |
| 0 | 09/30/2021 | Initial Publication of Section |
| 2 | 09/30/2022 | Minor wording updates |
| 3 | 08/01/2023 | Added new section 9-9 on Open Architecture (OA); Updated gradient references in 9-6 to include slope degrees and percentage |

## 9-1	Equipment Length and Width Considerations

New equipment and screening processes need to fit within the average checkpoint. Most of the equipment TSA procures is ultimately desired to hit FOC. As such, existing checkpoint conditions should be considered when developing new equipment and processes.

Length in the checkpoint is measured from the leading edge of divest, or the bin cart, to the trailing edge of the search table. The newest airport terminals in the federalized system can accommodate lanes that are longer than 70 feet, but most airports today cannot accommodate lanes of this length. Based on a recent sample set, approximately 20% of existing sites will fit screening lanes over 55 feet in overall system length; 50% will fit screening lanes based on 54 feet-56 feet in overall system length; 25% of existing sites will fit screening lanes based on 55 feet to 28 feet overall system length and 5% require a lane under 28 feet.

With regards to width, the equipment between the lanes where passengers traverse is known as the “infield.” Approximately 60% of infields range from 9 feet-8 inches to 14 feet with 20% under 9 feet-8 inches and 20% over 14 feet.

For other security equipment considerations outside of this space, guidance will be given during the Request for Proposal (RFP) process. The CRPG may be used as guidance during the qualification process (Qualification Data Package [QDP] and Functional Requirements Document [FRD]) as directed by the Program Office and the Requirements Development Branch.

| Design Examples | Examples of Common Design Limitations and Requirements |  |
| --- | --- | --- |

## 9-2	Equipment Maintenance Zones

Equipment maintenance zones should be carefully planned to accommodate current screening methods. Equipment in the infield must often be placed as close as possible to the AT or CT lane due to limitations in existing checkpoint widths. While overlapping maintenance zones can alleviate many issues, hard-mounted equipment is often in conflict and causes compromises that jeopardize the screening of passengers. OEM supplies should make all efforts to provide minimal and flexible maintenance zones. TSA shall have the right to place hard mounted equipment and barriers in the maintenance areas outside the main ATor CT body. AT or CT maintenance areas shall be limited to 24 inches. Areas outside the main body of the AT or CT may be limited to 2 inches of maintenance clearance for placement of AITs and barriers.

For other security equipment considerations, guidance will be given during the RFP process. The CRPG may be used as guidance during the QDP and FRD, as directed by the Program Office and the Requirements Development Branch.

## 9-3	Ceiling Height Limitations

The ceiling height standard for many airports is 8 feet above finish floor with elements, such as signage, that can extend down to 6 feet-8 inches. Because this height standard is common, it is imperative that dimensions are checked to ensure the equipment fits, including maintenance and other clearances, under an 8 feet high ceiling. The equipment shall also accommodate assembly and disassembly under the 8 feet requirement.

## 9-4	Equipment Interference Requirements

All TSA equipment shall be designed and engineered to not interfere with other TSE. Any zones of interference with existing equipment or new equipment in the TSA system shall be provided by the OEM prior to the certification process and must be discussed during the QDP process and/or demonstration. Test and evaluation may be required to determine interoperability of systems.

## 9-5	Structural Analysis

All equipment requiring seismic bracing in a public facility is governed by the International Building Code. The equipment shall have a structural analysis providing the information for the brackets and the equipment connection. The report shall be sealed and signed by a licensed structural engineer. Any equipment that poses a hazard to passengers due to collapse during a seismic event shall have a structural analysis of the body of the unit, including a finite element analysis, and shall be signed and sealed by a licensed structural engineer.

## 9-6	Sloped Floor Requirements

Ten percent of all checkpoints have sloped floors in some capacity. This is due to International Building Code allowing floors to slope up to 1:20 (i.e., 5% or 2.86 degrees) and not be classified as a ramp. In some cases, the slope may be uniform across an entire checkpoint. In other cases, a lane may be placed on flat surfaces that transition to a slope midway through the run of the lanes. Steeper slopes may require a different solution than shallower slopes, and minor slopes may be within operating tolerances and require no modifications. All TSE may be required to be designed and engineered to work on sloped floors up to a 1:20 (i.e., 5% or 2.86 degrees) slope. The equipment can be level with adjustable feet if passenger entrance and exit ramps do not exceed a 1:12 (i.e., 8.33% or 4.76 degrees) slope.

| Sloped Floor Study | Demonstration of CT Sloped Floor Resolutions |  |
| --- | --- | --- |

## 9-7	Equipment Geometry and Floor Load Drawings

The OEM shall provide equipment geometry drawings in the form of PDF and AutoCAD DWG shop drawing files. The OEM shall also provide a STEP (Standard for the Exchange of Product Data) file for import into AutoCAD version 2015 or later. The OEM shall provide a floor load and point load as well as floor loading directly under the footprint of the equipment. All drawings shall be in U.S. Standard measurement units.

## 9-8	Equipment Power and Data Coordination Drawings

The OEM shall provide power and data coordination drawings that clearly illustrate where power and data need to be provided at the unit for full functionality during the screening process. The OEM is required to be familiar with the current CRPG electrical provisions and provide information and drawings indicating any restrictions on type and location of devices at the unit. Cord lengths will need to reach from allowed power poke locations to the placement of the equipment. Minimum power cord lengths should be 20 feet. It is also critical to have uniform power plug configurations based on the type of equipment.

## 9-9	Open Architecture

TSA’s current security screening system is highly complex with limited data or interface standardization. Lack of standardization presents barriers to TSA achieving the desired security posture and hinders its ability to rapidly deploy innovative screening solutions to the field to respond to the evolving threat environment. It also increases the burden on our frontline officers who must perform their critical screening functions with cumbersome procedures, complex training, and varying user interfaces.

Open Architecture (OA) is a design approach in which equipment components, such as software and hardware, are standards-based and interoperable to allow a wide range of industry partners to create improved subcomponents (like new detection algorithms, user interfaces, or reporting systems). Leveraging this design approach will allow TSA to establish a superior transportation security System of Systems (SoS) that interact with each other and provide a unique capability that cannot be accomplished independently.

TSA’s vision is a connected transportation security SoS in which state-of-the-art solutions are quickly adopted to address emerging threats and enable a dynamic screening environment. This vision outlines our commitment to our most important asset, the dedicated frontline officers, securing the transportation system. Our vision seeks to provide the best tools to conduct mission critical screening operations while simplifying processes and procedures. This vision encompasses the combination of people, processes, and technologies across capability areas like identity management, accessible property screening, on-person screening, checked baggage screening, and alarm resolution. Planning efforts and designs should consider and incorporate the infrastructure requirements for the future concept of a connected checkpoint. Examples of these requirements may include a minimum of 10Gbps fiber networking infrastructure to send 1Gb DICOS files across the network in real-time, a fully redundant networking infrastructure to support failover, a dedicated and properly ventilated server room with access controls, sufficient space for one server rack per mod-set, sufficient power with room for expansion, proper IT physical security, etc.

# 10	Closing and Links to Appendices

Aviation security must continue to evolve and adapt to meet the constantly changing threat environment. SSCP designs, including electrical and data infrastructure, must incorporate current checkpoint design standards as well as the adaptability necessary to meet emerging threats and support emerging technologies. Improper SSCP design can result in terminal and checkpoint queue congestion, decreased efficiencies, flight delays, and unnecessary security risks. The information provided within the CRPG and the information in the appendices are sufficiently adaptable to include both current and future considerations. If you have any additional questions or comments, please contact Dale Mason (dale.mason@tsa.dhs.gov) Checkpoint Design SME.

Links to Appendices:

Appendix A provides equipment blocks and equipment tags and electrical device configurations for equipment, by technology and manufacturer. Appendix A may be found at https://www.tsa.gov/sites/default/files/checkpoint-requirements-and-planning-guide_appendix-a.docx.

Appendix B attempts to describe all electrical systems required to support the non-leased space at the checkpoint. Familiarity with these requirements is essential when designing a new checkpoint or reconfiguring an existing checkpoint. The power requirements for each individual piece of security screening equipment are included in the Power Requirements Table. Data requirements can be found in the electrical diagrams embedded with Section B-4. Appendix B may be found at https://www.tsa.gov/sites/default/files/checkpoint-requirements-and-planning-guide_appendix-b.docx

Appendix C illustrates multiple checkpoint layouts with different manufacturers’ equipment. These layouts serve as a guide for designing single-, two-, or three-lane checkpoints, as well as for placing equipment for multiple lane configurations. There is also example queue standards, remote screening, and STSO podium plans. Appendix C may be found at https://www.tsa.gov/sites/default/files/checkpoint-requirements-and-planning-guide_appendix-c.docx